June 20, 2000

Cellular Data Modem CM 900 (OEM)

Point-to-Point Wireless Access Solution.

# Table of Contents

- Product Overview						2

- Registering the CM 900 					3

- Starting and Configuring the CM 900			3

- Theory Of Operation						4

- Supported AT Commands					7

6)	Basic Trouble Shooting 					8

7)	Frequently Asked Questions (FAQs)				8

8)	Pin-out Diagram						10

Introduction

Product Overview

The Cellular Modem 900, CM 900, is a self-contained module that uses the Advanced Mobile Phone System (AMPS) cellular network for data transmission. AMPS is the cellular system used in North America, Latin America, Australia, New Zealand and parts of Africa. AMPS system is not compatible with the GSM system that is used in Europe and Asia.

The CM 900 has the choice of RS232 or TTL (V24) interface and supports the popular AT command set normally associated with modems.

##### CM 900 Features

The CM 900 provides the following features:

- OEM Module designed for embedded applications.

- Overall dimensions (excluding antenna) only 4.5 x 2 x 1.1 inches

- Uses AMPS Switched Circuit Cellular network for data transmission

- User friendly modem AT command set

- Choice of RS 232 or TTL (V24) serial interface

- Choice of Hardware or Software Flow Control

- Incorporates “Quiet Mode” (explained later) thereby enabling communication with “non-intelligent” systems

#### Sample Application Areas

Although any application requiring Data Communications can use the CM 900, the following are the areas where our OEM customers are using the CM 900.

- Packet Data Radio

- SCADA and Telemetry Systems

- Remote gas/water/electric meter readings.

- Factory Automation systems

- Vending Machines

Specifications

- Air Speed 						upto 9600 bps

- DTE Rate						300, 1200, 2400, 9600, 19200

57600 bps (DIP switch selectable)

- Data Compression & Error Detection		MNP2-5, V42, V42bis, MNP 10

- Transmit Power					0.6 watts

- Power Supply					5 Volts DC @ 1000 ma

- Interface						RS 232 / TTL (V24) Asynchronous

- Signals supported					TXD, RXD, CD, DSR, RTS, CTS, RI,

DTR

- Operating Temperature Range			0 to 70 deg C ( -10 to + 85 optional )

- Custom Configurations and Options		Yes

Your package includes:

- CM 900 unit (Radio included)

- Antenna

- Flat Cable (for RS 232 units only)

- This manual

What you need to provide:

- 5 Volts, 1.5 Amp DC  Regulated DC Power Supply

- Host PC with terminal program like Hyper terminal, Procomm etc

- MIN information (from your cellular service provider)

- SID Information (from your cellular service provider).

- A host PC with terminal program like Hyper terminal, Procomm etc

- Mobile Identification Number (MIN) provided by your Cellular service provider. MIN is your cellular phone number.

- System ID (SID) information (from your cellular service provider) SID uniquely identifies your cellular service provider.

Registering the CM 900 Unit

To register your CM 900 unit, do the following:

- Contact your cellular service provider

- Provide the cellular operator with the ESN (Electronic Serial Number) number located on the label at the top of your unit.

- Inform the cellular operator that the unit is an ONLY AMPS ANALOG unit with a power rating of class 3 (power 600 mw).

The cellular operator will register your unit and provide you the following information:

- MIN (Mobile Number) Number:

- System preference: A or B

- SID (System ID)

It is recommended that the MIN and SID information be stored in an easy to access location.

## Starting and configuring the CM 900 unit

The CM900 is configured as a DCE (e.g. modem). It communicates with a PC through the serial (RS 232) port.

RS 232 port			   DIP Switch

20 pin female header		      +   -  (5 volts Dc supply)

### View Component Side

## Starting the CM 900 Unit

## To start the CM 900 unit, perform the following steps:

- Connect the 5 Volts DC power supply. Please take precaution that the polarity of the applied voltage is correct. Connect the CM 900 to the PC with the RS 232 cable provided.

- Connect the Antenna to connector at the top left of the CM 900.

- Click the Hyper Terminal icon on the PC. Or select Start>Programs>Accessories>Hyper Terminal>Hyperterm.exe

The Connection Description Dialog Box opens.

- In the name field, enter CM 900 and click OK.

The Connect To dialog box is now active.

- Select Connect Using and then select the COM Port the CM 900 is connected to. For example, if the CM 900 is on COM 2, choose “Direct to Com2” and click OK.

- The COM2 Properties dialog box now opens. Select the following:

- Bits per second to match the DIP switch settings on the CM 900

- Data Bits: 8

- Parity: None

- Stop Bits: 1

- Flow Control based on the CM 900 switch settings.

- Click OK.

- Select Call>Disconnect

- Select Call>Connect

- Select File>Properties>Settings>ASCII setup

- Check the following:

- In the ASCII Sending field, check Echo typed characters locally

- In the ASCII receiving field, check Append line feeds to incoming line ends and Wrap lines that exceed terminal width.

- Click OK twice.

- Select Call>disconnect

- Select Call>connect

- Select File>Save

- Hit <CR> on the keyboard. The CM 900 will return OK.  This is the indication that the entire hardware setup and configuration is fine.

###### Configuring the CM 900

To configure the CM 900, perform the following steps:

- Enter AT+TEST<CR>. The unit responds OK.

###### Enter AT+MIN=< allocated_MIN ><CR> The unit responds OK.

- Enter AT+SID=< allocated_SID ><CR> The unit responds OK

- Enter AT+SYS=02 <CR>	(SID will determine preferred system, also called normal mode)

=00 <CR> (will select system B only)

=01 <CR> (will select system A only)

Check with your provider as to which is the recommended system.

The CM 900 does NOT have to configured every-time. The above information is stored in the unit’s non-volatile memory.

17)	Enter AT+MODE=02<CR> The unit is now set for data mode.

18)	Enter AT+SER <CR> The unit responds with SERVICE AVAILABLE.

Using the CM 900 to communicate

To use the CM 900 to communicate with a modem, perform the following steps:

- Enter ATDT XXXX <CR>

XXXX is the phone number of the modem you are calling. After approximately 30 seconds, the CM 900 responds with CONNECT message.

- Type “ Hello how are you”

The party you called should see the message on their terminal screen.

- Ask the called party to type a message and you should see that on your terminal screen.

- Enter the Escape sequence +++

The modem responds OK.

- Enter ATH0<CR>

The CM 900 responds  “No Carrier “ indicating that the connection is

terminated.

- Ask the remote party to dial you MIN number.

CM 900 responds with CONNECT message after approximately 30 seconds.

7)	Repeat steps 2 to 5.

## Theory of Operation

The CM-900 (hereafter also referred to as “modem”) is always in one of the two states:

- Command state

- Online state

This section describes the two states.

# Command State

The CM 900 assumes the Command state when it is turned ON. In this state, you can configure the unit for a particular application. In the Command State, all inputs from the PC are treated as commands.

## Command guidelines

Command line is a string of characters sent from the DTE (example PC) when the unit is in the Command state. All commands must adhere to the following guidelines

- Command lines must have a prefix , a body and a terminator.

- Command lines must begin with a character sequence AT.

- Command Lines must end with Carriage return (CR).

- Either all upper case or all lower case characters must be used.

- Each command must be entered separately.

- Command buffer is 40 characters.

Note: Commands entered incorrectly or with out-of-range parameters will result in ERROR message. Commands entered correctly will result in OK message.

## Result Codes

Result Codes refer to the responses of the CM 900 unit to a command.

Unless the CM 900 has been programmed to be in the Quiet mode (explained later in the AT command section), the unit will respond each time a command is issued. The default mode of the CM 900 is the Verbose mode. The responses in verbose mode are characters.

Using AT commands, the modem can be put into terse (non-verbose) mode. The responses in non-verbose mode are numerical digits.

Various modem responses are shown below:

Examples of CM 900 responses:

Response in			Response in		Meaning

Terse Mode			Verbose Mode

0				OK			Command executed without errors.

1				CONNECT		Connection at 300 bps

2				RING			Ringing signal detected

3				NO CARRIER		Carrier lost or never present

4				ERROR		Invalid command

5				CONNECT 1200	Connection at 1200 bps

6				NO DIALTONE	Cellular service not available

7				Busy			Called party on-line

8				No Answer

9				Connect 0600

10				Connect 2400

11				Connect 4800

12				Connect 9600

13				Connect 7200

66				Compression class 5		Connection established

MNP- 5

67				Compression V42bis

68				Not used

69				Compression none

76				Protocol: none

77				Protocol: LAPM

99				SERVICE UNAVAILABLE	Cellular service not available

100				SERVICE AVAILABLE	Unit ready for

communication

## On-line state

In online state, input(s) from the PC is treated as data to be shipped over the airwaves, and input(s) from the airwaves is treated as received data from the remote end.

The CM 900 is in online state in one of the two conditions:

- CM 900 dials out and a successful connection is established.

- CM 900 answers an incoming call and a successful connection is established.

When the CM 900 is in online state, all serial data presented to the radio is transmitted over the air.

You should NOT change communication parameters (e.g. baud rate) in this mode.

To go back to Command State, Escape Sequence (described in this section) is required.

## Transitioning from Online state to Command state (Escape sequence)

The sequence of characters to transition from Online State to the Command State is called the escape sequence.

For CM 900, he escape sequence is +++

The timing interval between the + signs must be between 250 msecs and 750 msecs.

When the CM 900 receives a valid escape sequence, the modem will enter Command State and respond with OK.

## Supported AT commands

This section describes the basic Attention (AT) commands used to configure the CM 900. These commands are a subset of the commands used in typical high-speed modems.

A command line is a string of characters sent from the DTE (e.g. a PC) while the modem is in Command State. A command line has a prefix, a body and a terminator. Each command line must begin with a character sequence AT and must terminate with a carriage return.

The Radio starts the processing of commands only after AT string sequence is received.

Commands entered with out of range parameters will result in ERROR message.

Correct commands will result in OK message in verbose mode, and 0 in terse mode.

Supported AT commands:

Command			Action 				 		Note

1)	ATZ<CR>			Resets the modem.

The unit responds with OK messages

2)	ATO<CR>			Go on line. This is done after

the state of the unit is changed from

on-line to command state.

ATO then puts the unit back into

online state.

3)	ATDT XXXX <CR>		Cell modem dials out.

XXXX is the phone number

4)	AT+TEST <CR>		Puts the unit in test mode

The commands AT+SYS, AT+MIN, AT+SID and AT+ID can be entered only when the unit is in Test Mode.

5)	AT+SYS=xx<CR>		Sets system preference.

SYS =00 sets the unit for system B only

SYS =01 sets the unit for system A only

SYS =02 is for normal mode. SID determines preferred system

SYS =03 is for home use only

6)	AT+RSSI<CR>		Indicates the Received Signal

Strength. Should be > -100 dbm for

reliable communication.

7)	AT+SER <CR>		Indicates if service is available or not

8)	AT+MIN=XXXXX<CR>	Sets the MIN number. The modem responds OK

9)	AT+SID=XXXX<CR>		Sets SID. Modem responds OK

10)	AT+ID?<CR>			Displays ESN, MIN, SID,

System Preference information.

Note: MIN, ESN, SID information are stored in the non-volatile memory of the cell-modem. These do not have to be entered every-time on power-up.

To exit test mode, enter AT+MODE=02<CR>

11)	ATQ1<CR>			Unit does not respond with OK or non-verbose

messages. This is ideal for in situations where the Data Link is connected to “non-intelligent” host.

This mode is also called Quiet Mode.

12)	ATQ1&W&W1		Q1 instruction stored in non-volatile memory.

13)	AT&V<CR>			Displays settings of the cell modem

14)	ATI3<CR>			Displays Software Revision #

15)	ATH0				Modem disconnects and initializes

16)	ATV0				Unit responds in terse mode. Results code

Displayed as digits

17)	ATV0&W&W1			Above command stored in non-volatile memory.

On power-up, the modem will be in terse mode.

18)	ATV1				Unit responds in verbose mode. Results codes

are displayed as messages.

19)	ATV1&W&W1			Above command stored in non-volatile memory.

On power-up, the modem will be in verbose mode.

20)	ATS0				Default is 0. If 0, then the unit will not answer

incoming call. Please use ATS0=1&w&w1 to have

the unit answer incoming calls.

Escape Sequence:

When the modem is on-line state, it is possible to break the data transmission/reception.

This is achieved by sending a sequence of three ASCII characters. For ease of use, the default character is +.

The Radio will respond with OK on detection of valid escape sequence.

21)	+++		Escape sequence		OK

## Basic Troubleshooting :

- If the unit draws too much power. Please ensure that the correct power supply is used and correctly wired.

- If dialing out is repeatedly unsuccessful and modem gives the message “NO DIALTONE ”, please check if service is available.

- If the unit never connects on dialing out, please ensure that the ESN (Electronic Serial Number) and the MIN numbers are right.

- Please check RSSI level. If low (lower than –95 dbm), placing the antenna vertical will help.

- If remote landline modem connection drops frequently, please have the calling party set S7=60 & S10 register to 200. This will make the modem more tolerant of noise hits. This can be done as follows:

ATS0=1S7=60S10=200&W&W1<CR>

- If for some reason, the CM 900 unit is not responding, please try the command AT&F&W&W1 <CR>

Whenever the command AT&F&W&W1 is issued, then you must do the following

ATS0=1\N2+MS=11,1,1200,2400&W&W1 <CR>

AT-SEC=1,15&W&W1 <CR>

- If that also fails, power down the unit and power it up again.

8)	Please read next section on FAQs.

Frequently Asked Questions (FAQs)

1)	Question:	What is the flow control method recommended?

Answer:	If the host is a PC, Hardware flow control is recommended.

However, in most control instrumentation, either software or no flow control is called for.

2)	Question:	How do I disable any flow control?

Answer:	Simply disable both the Hardware and software control switch settings. This is not recommended for large file transfers.

3)	Question:	I need to connect the Cellular Data Link to a machine which

does not understand your messages. All it needs is data. Which

configuration is recommended?

Answer:	Please use the ATQ1&w&w1<CR> command.

4)	Question:	Have you experienced that fact that communication with certain

brands of modems is unreliable?

Yes. And more so with generic brand which tend to throw hot

signals (i.e strong signal levels) on the line. This causes the

Cellular service to over-modulate which leads to distortions and

results in unreliable connections.

5)	Question:	What is the solution then?

Answer:	We recommend that you use modems that support MNP-10

and also allow transmit level to be reduced to between –15 dbm.

and –20 dbm. Rockwell based modem designs generally meet

these requirements.

For Rockwell based modems, please try the following commands

on the land line modem:

AT-SEC=1,18 <CR>. This will enable MNP 10 and set transmit

level to –18 dbm.

AT+MS=10,1,1200,4800&W&W1<CR>. Sets the modem to communicate between 1200 and 4800 bps.

ATS91=15 <CR>.

6)	Question:	Is there any simple way to avoid frequent loss of connection?

Answer:	Please program the CM 900 as follows

AT+MS=2,1,1200,1200&W&W1<CR>

This will force the CM 900 to connect only at 1200 bps.

7)	Question:	Can IDC provide a landline modem I can use?

Answer:	Yes.

8)	Question:	What initialization string is recommended to communicate with

non-MNP modem?

9)	Answer:	Connection with non-MNP landline modems is not recommended.

10)	Question:	My requirement(s) are totally unique. Can I count on IDC to help?

Answer:	Absolutely.

11)	Question:	Is there any recommended init string for various land line

modems?

Answer:	the following have been tried with great success:

- For US Robotics / 3 Com modems:

ATS10=200&A3&B1&N2&U2&W&W1 <CR>

- For Rockwell chip set based land-line modems 33600 and lower

AT%E2\N2S7=60S10=200+MS=11,1,1200,4800&w&w1 <CR>

- For most other brands:

ATN0S37=6S10=200&w&w1 <CR>

- For Rockwell chip set based land-line modems 56K V90 models AT%E2\N2S10=200S7=60+MS=V34,1,1200,2400,1200,2400 <CR>

RS 232 pin-out diagram for the CM 900

Dip Switch

300 bps

1200 bps

S/W flow cntrl

6

Gnd

Vcc

1

5

10 pin header

10

20 pin header

View Solder Side

GND	6			1	Power / VCC (Square Pad)

DTR	7			2	RI

TXD	8			3	CTS

RXD	9			4	RTS

DCD 	10			5	DSR