ProComm (tm)

Version 2.2

Program Reference Manual

February 21, 1986

PIL Software Systems

P.O. Box 1471

Columbia, MO 65205

BBS: (314) 449-9401

Copyright (c)  1986 PIL Software Systems, Bruce Barkelew and

Tom Smith. All Rights Reserved.

COPYRIGHT

This program  is  copyright  (C)  1985, 1986 by PIL Software

Systems, Bruce Barkelew and Tom Smith. All  rights reserved.

Non-registered users are granted  a limited  license  to use

this  product on a trial  basis, and to copy the program for

trial use by others subject to the following limitations:

The program is distributed in unmodified form, complete

with documentation.

No fee, charge or  other consideration  is requested or

accepted.

The program is not  distributed in conjunction with any

other product.

If you intend to  use  ProComm  on a  regular  basis, please

register and  show your support for the authors.  Commercial

or business use by non-registered users is prohibited.

Interested in  multiple  copies for  use at  work? Site  and

corporate licenses are available. We'd be happy to work with

you to solve your business communications needs.

TRADEMARKS

ProComm is a trademark of PIL Software Systems.

Most of the hardware names in this  manual are trademarks or

trade names of specific manufacturers.

Copyright (C) 1986 PIL Software Systems                   ii

ProComm 2.2                                         Contents

CONTENTS

Preface to Version 2.2  . . . . . . . . . . . . . . . . .  1

Registration  . . . . . . . . . . . . . . . . . . . . . .  2

User Supported Software . . . . . . . . . . . . . . . . .  3

Product Support . . . . . . . . . . . . . . . . . . . . .  4

Introduction  . . . . . . . . . . . . . . . . . . . . . .  6

Program Description . . . . . . . . . . . . . . . . . .  6

Machine Requirements  . . . . . . . . . . . . . . . . .  6

ProComm Files . . . . . . . . . . . . . . . . . . . . .  7

Program Features  . . . . . . . . . . . . . . . . . . . .  8

Starting the Program  . . . . . . . . . . . . . . . . .  8

Terminal Mode . . . . . . . . . . . . . . . . . . . . .  9

Terminal Emulations . . . . . . . . . . . . . . . . .  9

Communication Parameters  . . . . . . . . . . . . . . .  9

Output String Translation . . . . . . . . . . . . . . . 10

Setup Screen  . . . . . . . . . . . . . . . . . . . . . 11

Modem Setup . . . . . . . . . . . . . . . . . . . . . 12

Terminal Setup  . . . . . . . . . . . . . . . . . . . 15

Kermit Setup  . . . . . . . . . . . . . . . . . . . . 16

General Setup . . . . . . . . . . . . . . . . . . . . 16

Dialing Directory . . . . . . . . . . . . . . . . . . . 19

Searching for an Entry  . . . . . . . . . . . . . . . 20

Revising the Dialing Directory  . . . . . . . . . . . 20

Deleting Entries  . . . . . . . . . . . . . . . . . . 22

Making a Call . . . . . . . . . . . . . . . . . . . . 22

Printing the Directory  . . . . . . . . . . . . . . . 22

Automatic Redial With Circular Dialing Queue  . . . . . 22

File Transfer . . . . . . . . . . . . . . . . . . . . . 23

Uploading Files . . . . . . . . . . . . . . . . . . . 23

Downloading Files . . . . . . . . . . . . . . . . . . 24

File Transfer Protocols . . . . . . . . . . . . . . . 24

Translate Table . . . . . . . . . . . . . . . . . . . . 27

Command Files . . . . . . . . . . . . . . . . . . . . . 27

Keyboard Macros . . . . . . . . . . . . . . . . . . . . 36

DOS Gateway . . . . . . . . . . . . . . . . . . . . . . 36

Editor  . . . . . . . . . . . . . . . . . . . . . . . . 37

Screen Dump . . . . . . . . . . . . . . . . . . . . . . 37

File and Printer Logging  . . . . . . . . . . . . . . . 37

Setting Display Colors  . . . . . . . . . . . . . . . . 38

Redisplay Buffer  . . . . . . . . . . . . . . . . . . . 38

File Directory  . . . . . . . . . . . . . . . . . . . . 39

View a File . . . . . . . . . . . . . . . . . . . . . . 39

Program Information . . . . . . . . . . . . . . . . . . 39

Change Logged Drive or Directory  . . . . . . . . . . . 39

Display Elapsed Time  . . . . . . . . . . . . . . . . . 39

Clear Screen  . . . . . . . . . . . . . . . . . . . . . 40

Hang-up . . . . . . . . . . . . . . . . . . . . . . . . 40

Copyright (C) 1986 PIL Software Systems                  iii

ProComm 2.2                                         Contents

Toggle Duplex . . . . . . . . . . . . . . . . . . . . . 40

Toggle CR - CR/LF . . . . . . . . . . . . . . . . . . . 40

Break . . . . . . . . . . . . . . . . . . . . . . . . . 41

Ending a ProComm Session  . . . . . . . . . . . . . . . 41

Appendix A - Terminal Emulation . . . . . . . . . . . . . 42

Appendix B - Command Reference Guide  . . . . . . . . . . 53

Appendix C - Command File Commands  . . . . . . . . . . . 54

Appendix D - ProComm Utility Programs . . . . . . . . . . 57

Appendix E - Answers to Commonly Asked Questions  . . . . 58

Appendix F - ProComm Technical Specifications . . . . . . 61

Appendix G - ProComm 2.2 Invoice  . . . . . . . . . . . . 62

Index . . . . . . . . . . . . . . . . . . . . . . . . . . 63

Copyright (C) 1986 PIL Software Systems                   iv

ProComm 2.2                                          Preface

PREFACE TO VERSION 2.2

----------------------

Changes  to version 2.2 are  summarized in  the accompanying

file PRCM22.NEW. This manual has been updated to reflect the

current status of the product.

One major change worth mentioning here  is the switch to  an

overlay  structure.  This  switch,  coupled  with  extensive

enhancements, has resulted in an executable file quite a bit

larger than  that  of  previous  versions.  However,  actual

memory requirements have been reduced.

Once again the parameter file has also increased in size. If

you are running version 2.1 the .PRM file will be  converted

to the  2.2  format with  all  of  your  previously  defined

options intact. If you are using earlier versions of ProComm

a completely new parameter file will be created.

Quite a number of script file commands have been changed. It

is advised  that  you  check all  previously created command

files before relying upon them for unattended operation.

In  addition,  several  keystrokes  for commands  have  been

changed,  most  notably  in  the  terminal   emulations.  We

apolgize for the inconvenience this  causes, but  we felt it

was justified in terms of overall funtionality.

Copyright (C) 1986 PIL Software Systems                    1

ProComm 2.2                                     Registration

REGISTRATION

------------

User feedback is an integral part of any successful software

system. Please share your impression  of ProComm by  leaving

us a message  on our support bulletin board system with your

name, address,  occupation  and  any comments or suggestions

you care to pass along. You might also mention where you got

the program, and how you intend to use it. Call  us at (314)

449-9401.  The BBS runs 23 hours a day (4:00am to 3:00am), 7

days a week.

ProComm  is  distributed  as   User-Supported  Software.  We

encourage you to  try the program and share  it with friends

as long as:

The  program  is  distributed  complete and  unchanged,

accompanied by the documentation.

No fee, charge or compensation is requested or received

except by PIL Software Systems.

ProComm  is  not distributed  in  conjunction  with any

other product.

If you use our program on a regular basis, please show  your

support by registering. You may register by sending  a check

or money order for $25 to:

PIL Software Systems

PO Box 1471

Columbia, MO 65202

Registered users will receive notification of major releases

of ProComm. They will also receive priority treatment on our

support  bulletin   board  system,  and  access  to  various

restricted files, programs and information.

If you or your company needs an invoice in order to generate

payment, one is included in this manual as appendix G.

Copyright (C) 1986 PIL Software Systems                    2

ProComm 2.2                          User Supported Software

USER SUPPORTED SOFTWARE

-----------------------

User-supported  software  is   a  means  for  the  computing

community  to  receive   quality   software  while  directly

supporting software authors. It is based on the ideas that:

The value and utility of  software  is best assessed by

the user  on his or her own system.  Only after using a

program can  one  really  determine  whether it  serves

personal applications, needs and tastes.

The creation of independent  personal computer software

can and should be supported by the computing community.

Copying  of  programs should be encouraged, rather than

restricted.  The   ease  with  which  software  can  be

distributed  outside  traditional  commercial  channels

reflects the  strength, rather than  the  weakness,  of

electronic information.

Under the user supported concept, anyone  may request a copy

of  a user-supported program by sending a  blank,  formatted

disk  to  the  program author  together  with  an addressed,

postage-paid  return mailer. A copy  of  the  program, along

with documentation,  will be sent  by  return  mail  on  the

user's disk.

The program carries a notice suggesting registration for the

program. Registration is voluntary on your part.

Regardless of  whether  you register,  you are encouraged to

copy   and  distribute   the  program   for   the   private,

non-commercial use of others.

Copyright (C) 1986 PIL Software Systems                    3

ProComm 2.2                                  Product Support

PRODUCT SUPPORT

---------------

In order  to support  our users,  and to provide a  means of

distribution, we  have  implemented  a bulletin board system

you can use to communicate with us. If you have questions or

comments just  call us  up  and leave a note. In your  note,

describe  as completely as  possible  the  problem  you  are

having. Let us know your machine configuration, your ProComm

configuration and version and any resident software you  are

using. Describe  what  steps you  take  before  the  problem

occurs, and exactly what the program does when it occurs. If

you  do not provide us  with  a  complete description of the

problem there is little we can do to help. We'll do our best

to keep you up and running, but if you are  not a registered

user we do not guarantee to provide support of any kind.

You'll  always be able to find the latest version of ProComm

on the  BBS. The number for our board is (314) 449-9401. The

board is operational 23 hours per day, 4:00am-3:00am CST. We

are using the Fido BBS Network system from Tom Jennings. You

can  send  mail to us via FidoNet at  node #619,  region #14

(14/619).

In addition  to our board, several fine systems always carry

the latest version of ProComm. They are:

Mike Johnson's Computer Aide BBS, Fido #19/329

Tulsa, OK. (918) 493-2137

2400 baud, 24 hours a day

John O'Boyle's PConnecticut BBS

West Hartford, CT. (203) 521-1991

2400 baud, 24 hours a day

Sal Manaro's Underdog BBS

Seattle, WA. (206) 725-9233

2400 baud, 24 hours a day

Ken Shackelford's ATL/FIDO1, Fido #18/1

Woodstock, GA. (404) 928-1876

2400 baud, 24 hours a day

Dave Staehlin's TBC BBS

Albuquerque, NM, (505) 821-7379

2400 baud, 24 hours a day

Dan Kardell's Rowlett RBBS

Rowlett, TX, (214) 475-4598

2400 baud, 24 hours a day

Steve Clancy's WELLSPRING BBS

Copyright (C) 1986 PIL Software Systems                    4

ProComm 2.2                                  Product Support

Irvine, CA, (714) 856-7996

1200 baud, 5pm-8am M-F, 24 hours weekends

Larry Trost's Buccaneers Cove TBBS

San Jose, CA, (408) 370-9187

2400 baud, 24 hours a day

Mike Todds IBM/PC SIG

The Source

ProComm  is also found  on  a host of  other  bulletin board

systems, user groups and the like.

Copyright (C) 1986 PIL Software Systems                    5

ProComm 2.2                                     Introduction

INTRODUCTION

------------

ProComm  is a general purpose communications program with  a

wealth  of  features.  It  was  written  in  "C"  with  some

assembler  routines  for  optimum  performance. We hope  you

enjoy using it as much as we enjoyed writing it.

PROGRAM DESCRIPTION

-------------------

ProComm was designed to provide pleasant  access to the kind

of tasks we  thought a communications  program should offer.

Included in its features are:

The ability to emulate a number of popular terminals.

A dialing directory containing one hundred entries.

Auto  redial facilities  for connecting  with  hard  to

reach numbers.

Several  popular  file  transfer  protocols   including

XMODEM, Kermit, Telink and more.

Command files to control automatic logon and unattended

operation.

A  DOS gateway which allows you to execute DOS commands

or other programs while you're still on-line.

And a host of other goodies including keyboard  macros,

disk and printer logging and much much more.

MACHINE REQUIREMENTS

--------------------

ProComm requires 128K of available RAM  to execute properly.

That's  128K in addition  to  the operating system  and  any

resident  programs. It runs  under MS-DOS on the IBM PC, XT,

AT or  any close  compatible. It may  be used with either  a

color, composite or monochrome display.

ProComm may have a number of files open at once so make sure

that the  FILES parameter  in  your CONFIG.SYS file is large

enough. We  suggest  using FILES=15 or greater. Consult your

MS-DOS manual for a discussion of the CONFIG.SYS file.

Copyright (C) 1986 PIL Software Systems                    6

ProComm 2.2                                     Introduction

PROCOMM FILES

-------------

ProComm creates and uses several files. They are:

PROCOMM.PRM     Default parameter file

PROCOMM.DIR     Dialing directory file

PROCOMM.KEY     Keyboard macro file

PROCOMM.XLT     Translate table file

ProComm  uses the environment variable  PROCOMM= to  tell it

where  to look  for its  files. It  will  first  search  the

current directory  for the needed file. If it  is not there,

ProComm will search the directory pointed to by PROCOMM=. If

it is  found in  neither place, and needs  to be created, it

will be created in the  directory pointed to by PROCOMM=. If

the environment  variable is not  set, files will be created

in the current directory.

You may set this variable by issuing the DOS command:

SET PROCOMM=pathname

where  'pathname' is a fully qualified path name ending with

a  backslash.  You may  issue  this command  from either the

command line or from a batch file. For instance, if you have

the  command SET PROCOMM=C:\COMM\PROCOMM\  in  your autoexec

file,  ProComm will  know  to  look  for  its  files  in the

C:\COMM\PROCOMM  directory and  you can use the program from

anywhere on  your  system and  have just one  set of ProComm

files.

Clear the environment variable by saying:

SET PROCOMM=

Consult  your   MS-DOS  manual  for   more   information  on

environment variables.

Copyright (C) 1986 PIL Software Systems                    7

ProComm 2.2                                 Program Features

PROGRAM FEATURES

----------------

STARTING THE PROGRAM

--------------------

Begin a ProComm session by issuing the command:

PROCOMM [/S] [/B] [/Ffilename] [/M]

Command line switches may appear in  any  order, but must be

separated by at least one blank.

Including  a '/S'  on  the command line  suppresses  ProComm

produced sound effects. It does not affect beeps (^G) coming

from the remote computer or the alarm function,  nor does it

control the modem speaker. You can change the  default sound

setting using the setup (ALT-S) screen.

The '/B' option may be used to run the program in black  and

white.  This is useful  for users  with  composite  monitors

attached  to color  graphics  display cards.  If the '/B' is

omitted ProComm will automatically  adjust itself for either

color or  monochrome  operation.  To  make  black  and white

operation  the default, begin  a ProComm  session using  the

'/B' option.  then save the current  colors using  the ALT-Z

facility.

Specify  '/F' with  a filename  to execute  a  command  file

immediately  upon  program   startup.  Command   files   are

described later in this chapter.

The '/M'  option sets  ProComm to  perform  screen  displays

using BIOS  calls, rather than direct screen writes. This is

useful for  running  under  multi-tasking operating systems.

Screen write method  is  also selectabel  through the  setup

facility.

When  the program  begins it displays  the ProComm logo  and

copyright  notice,  and  possibly  the  program  information

screen. This brief commercial occasionally  presents  itself

the  first few times  you  use  the  program.  You  can also

display the information screen using the ALT-I command.

Copyright (C) 1986 PIL Software Systems                    8

ProComm 2.2                                 Program Features

TERMINAL MODE

-------------

After the opening  display  and the  initialization  of  the

program,  you  are  left  in  terminal mode.  Most  of  your

communicating will  take  place here. The bottom line of the

screen is  reserved  for messages but the rest of the screen

is  open  for  use. Most ProComm  features are  called  from

terminal mode. They are usually executed from a window which

leaves your original screen intact. Press ALT-F10 for a list

of  available  commands.  You  may  then  select  an  option

directly from the help screen.

Terminal Emulations

-------------------

Most  keyboard and screen functions  will operate locally as

you expect them  to.  What  they  do  on the remote computer

depends on which  terminal emulation you are  using. ProComm

currently supports ten popular terminal configurations:

- IBM 3101

- DEC VT-100

- DEC VT-52

- Televideo 910/920

- Televideo 925/950

- Lear Siegler ADM-3/5

- Heath/Zenith 19

- ANSI-BBS

- ADDS Viewpoint

- Wyse 100

To change the emulation  in use,  select TERMINAL SETUP from

the setup screen (ALT-S).

Consult appendix A for a list of the functions supported for

each terminal.

COMMUNICATION PARAMETERS

------------------------

ProComm supports a variety of communications configurations.

You  may select baud rate, the number of data and stop bits,

parity  and  duplex.  The  program  defaults   to  300  baud

operation, with 8  data bits, 1 stop bit, no parity and echo

off (full duplex).

Copyright (C) 1986 PIL Software Systems                    9

ProComm 2.2                                 Program Features

Use  the ALT-P  command to review or change  the active line

control   parameters.  After   opening   the   Communication

Parameter  window, ProComm  displays  the current  settings.

Enter  a number from  1  to 12 to  change  to  a  predefined

setting. Use  the numbers  13 thru  19 to setup a customized

configuration. Enter 20 to select COM1  as your active port,

or 21 to use COM2.

Use the save option (22) to make the  new configuration your

default. If you change the parameter settings without saving

them  to disk,  they  will  only  be  used  for the  current

session.

Press ESC to return to terminal mode. The new parameters are

now in affect.

OUTPUT STRING TRANSLATION

-------------------------

There  are several situations in which ProComm will  perform

translation on strings sent to  the modem. These include the

modem  initialization string,  the  modem  command,  dialing

directory  numbers,  long  distance  codes,  keyboard  macro

strings,  and  the  %T,  %N  and  %J  script  commands.  The

translation allows you to send carriage returns, escapes and

other control characters to the  modem, and  also provides a

pause feature.

ProComm  manages this  by providing  translation  characters

which   can  be  specified  in  the  general  setup  screen.

Characters are provided for CR, ESC, CTRL-char and pause.

The  default CR  translation  character  is  the exclamation

point  ('!').  Anytime  that  ProComm   encounters   the  CR

translate character in one of the above mentioned strings it

replaces  that  character with a CR (ASCII 13). To  send the

character without translation use two of them together.  For

instance, in the default case where  '!' is the CR translate

character, a keyboard  macro set  to 'PASSWORD!'  sends  the

string 'PASSWORD'  followed by a carriage return. A keyboard

macro  set to 'GO AWAY!!' will be translated  as 'GO AWAY!'.

Since two of  the characters  in a row are translated as the

true character, some other  method is necessary to  send two

adjacent  carriage  returns.  You  can  do  this  using  the

CTRL-char translation  described below. For instance, if the

CTRL translate character  is  set  to the  caret  ('^')  the

string '^M^M' is  translated as  2 carriage returns (because

CTRL-M is a CR).

The  default ESC  translation character  is the vertical bar

('|').  Anytime that  ProComm encounters  the ESC  translate

character in one of the above mentioned strings  it replaces

Copyright (C) 1986 PIL Software Systems                   10

ProComm 2.2                                 Program Features

that character with an ESC (ASCII 27). To send the character

without  translation use two of them together. For instance,

in   the  default  case  where  '|'  is  the  ESC  translate

character,  a keyboard  macro set to '|[2J'  sends the VT100

command  for clear screen (ESC [ 2 J). A  keyboard macro set

to '|| box ||' will be translated as '| box |'. Since two of

the  characters   in  a  row  are  translated  as  the  true

character,  some  other  method is  necessary  to  send  two

adjacent  escapes.  You  can  do  this  using the  CTRL-char

translation  described  below. For  instance,  if  the  CTRL

translate  character is  set  to the caret ('^') the  string

'^[^[' is translated as 2  escapes  (because  CTRL-[  is  an

ESC).

The  CTRL  character translation is  slightly  different. In

this  case  the  CTRL  translate  character  is  a  lead  in

indicator that the following character should be interpreted

as  a CTRL character. For instance, using  the  default CTRL

translate character '^' the string '^C' would be interpreted

as  a  CTRL-C  (ASCII  3).  Again,  2  translate  characters

together are treated as the actual character, so '^^C' would

be translated  as  '^C' (the  caret character  followed by a

capital C).

The final translate character provided is the pause. If this

character  is encountered  during translation,  ProComm will

pause for 1/2 second before doing anything else. The default

translate pause character  is the tilde  ('~'). This  one is

different  from the  other translate characters  in  that  2

pause characters  together cause  a 1 second pause; they are

NOT interpreted as  the  single  character. If  you  wish to

actually  send that character you must change  the setup for

the pause translate character.

Please  note  the  difference  between  the translate  pause

character and a modem  pause character.  The translate pause

character causes a  pause in  characters being  sent  to the

modem. This is appropriate for pauses around a modem  escape

code, as  in a hang-up string,  or  to  allow the  modem  to

respond  to  something  before  continuing.  A  modem  pause

character, such  as  the comma used by Hayes, causes a pause

in characters sent from  the  modem to the phone line. It is

appropriate  for waiting for  second dial tones and such, as

when  you  are  dialing out  through a PBX  or office  phone

system. The two are NOT the same and should not be confused.

SETUP SCREEN

------------

ProComm allows you  to  define  many of  your  own power  up

defaults  for  system  parameters.  You  can also  change  a

setting temporarily, and  go back to  your defaults for  the

Copyright (C) 1986 PIL Software Systems                   11

ProComm 2.2                                 Program Features

next  session.  To  access the setup  routine,  press ALT-S.

Setup is currently  divided  into 4  sections:  modem setup,

terminal setup, Kermit setup and general setup.

Select a section by typing  its number  and pressing return.

You will be presented  with a screen containing  the current

defaults for that section.  To change a  setting,  enter the

number of the parameter followed by  a  CR.  Then  make your

change.  If, after selecting a parameter, you  decide not to

change it,  simply  press ESC. The current value will remain

unchanged.

If the  parameter chosen requires a  string, simply  type in

the  characters for the  new  string followed  by a carriage

return. Pressing CR  without typing any  characters will set

the string to null.

Some parameters offer you a selection of valid values. Press

the space bar to  rotate between values.  When the value you

desire is displayed, press CR to accept it.

If the parameter  requires a  number,  enter the number  and

press return.  ProComm will  check to  make  sure  that  the

number falls within a valid range.

To exit a setup section, press ESC. If  you wish to save the

changes  you have to  disk,  select option 5  from  the main

setup option list.  If you  select  this option the  current

settings will  become your  power-up default. If  you do not

save the setup, the changes you have made will be in  effect

for only the current ProComm session.

To exit the main setup screen press ESC.

Modem Setup

-----------

Modem Initialization String

This is  the string sent to the modem every time you begin a

ProComm session. It  may be up  to 46 characters  in length,

and may contain carriage returns or control characters using

the translate conventions described in the section on output

string  translation.   Modem  initialization   strings  vary

between different types of modems. The default is configured

for  a Hayes  or compatible modem. It is 'AT E0 S7=60 S11=55

V1 X1', where

AT      is the command prefix

E0      sets modem echo of commands off

S7=60   wait max 60 seconds for carrier

Copyright (C) 1986 PIL Software Systems                   12

ProComm 2.2                                 Program Features

S11=55  touch tone spacing (fast)

V1      verbal result codes

X1      extended result codes

The initialization  string  may  be  tailored  to  suit your

particular  modem and  needs. Consult your  modem manual for

details regarding  the operation and available commands  for

your modem.

Dialing Command

The dialing command is  used to instruct the modem to dial a

number.  It is  sent  to  the modem  by  the dial  or redial

functions,  followed  by  a  number  to  dial and  any  long

distance codes  you  might  specify. The default  is  'ATDT'

where:

AT      is the command prefix

D       is the dial command

T       turns on touch tone dialing

The modem  dialing command may contain imbedded pauses, CRs,

ESCs and CTRL  characters using the conventions described in

the section on output string translation.

Connect String

This is the message  your modem sends to ProComm to indicate

that  a   connection  has  been  achieved.  It  defaults  to

'CONNECT'.  ProComm uses  this  value during auto  redial to

determine  when  a connection has been  made. Note that this

parameter must be  set correctly  (including  case) for  the

redial (ALT-R) feature to work.

Modem No Connect Strings

These strings are also used by the auto redial feature. They

should  be  set to  modem  messages  that  indicate  that  a

connection  was  not  achieved.  The  defaults  are  'BUSY',

'VOICE' and  'NO CARRIER'. Again, they must be exact matches

to  the messages your modem  displays, including case.  Some

modems  do   not   support  these  extended  call   progress

monitoring features.

Hang-up String

The hang-up string is the command to be sent to the modem to

cause it to hang up. ProComm first  attempts to  hang-up  by

dropping the Data Terminal Ready (DTR) line. If that is  not

successful  it  will  send  this string  to  the modem.  The

default,   set   up   for   Hayes   and    compatibles,   is

'~~~+++~~~ATH0!', where:

~~~     is a 1 1/2 second delay

Copyright (C) 1986 PIL Software Systems                   13

ProComm 2.2                                 Program Features

+++     is the modem escape sequence

~~~     is a 1 1/2 second delay

AT      is the modem command prefix

H0      is the hang-up command

!       sends a CR

The '~'  and '!'  characters are translated  by ProComm, not

the modem  (see the section on output  string  translation).

The '+++'  surrounded by the 1  1/2 second pauses causes the

modem to go into command state, where the hang-up command is

then issued. For more  information  see  the section on  the

ALT-H command.

Redial Timeout Delay

This is the number of seconds that ProComm will wait, during

a redial,  for a  connect  or no  connect response  from the

modem.  If the  timeout  delay is exceeded  before  a  modem

response is received, ProComm will cycle and attempt another

redial. This value may  also be modified on the fly during a

redial attempt.  See  the  section  on  re-dialing for  more

details.

Redial Pause Delay

The redial pause delay is the number of seconds that ProComm

will  pause  between  each dialing attempt during  a redial.

This  is needed  by some modems to reset  between calls. The

default, and the minimum pause time, is 1 second.

Modem CD Mask

The  modem  Carrier  Detect  mask  is  used  by  ProComm  to

determine whether or not a hang-up has  been successful. The

default  value  is  32.  It  may  be necessary  for  you  to

experiment  with  this  parameter in order  to determine the

correct  value.  Values  tried  should  powers  of  2,  i.e.

16,32,64 and 128. In general, if the hang-up command (ALT-H)

correctly reports the results of  the  hang-up attempt, then

the  CD  mask  is   correct.  If  the  hang-up  attempt  was

successful, but reports that the call  was not disconnected,

or vice versa, then the CD mask is  incorrect. The following

values may be used as guidelines, although cabling and other

considerations may make them inaccurate:

16      Hayes external, Prentice POPCOM

32      USR Courier, Cermetek Security Modem

128     Hayes internal, Qubie, USR Password

Rixon R212A

See  the  section on  the hang-up  (ALT-H) command for  more

details.

Copyright (C) 1986 PIL Software Systems                   14

ProComm 2.2                                 Program Features

Terminal Setup

--------------

Terminal Emulation

Use  this parameter  to  select  the  terminal  you wish  to

emulate. After selecting option 1, press the space bar until

the  terminal desired is displayed. Then  press  CR  to make

that  the  current emulation. Take care to  insure that  the

rest  of  the  settable  terminal  parameters match what  is

expected  for the terminal type you select.  For general BBS

work ANSI-BBS is suggested.

Duplex

Control the default duplex setting here.  Your  choices  are

full duplex (no echo) and half duplex (local echo).  If what

you  type is not displayed  on  the  screen, try using  half

duplex. If  you see  two  of every  character you type, give

full duplex a shot.

Handshake

There is currently  only 1  handshake  that  is available to

ProComm users:  XON/XOFF. If you desire  to  use handshaking

set this  option to XON/XOFF, otherwise select NONE. ProComm

performs  XON/XOFF  at  the   interrupt  level.  To  prevent

deadlock due  to  extraneous  XOFFs  caused  by  line  noise

ProComm allows you to reset the XOFF flag. If you ever see a

message on  the  status  line  that  indicates  an XOFF  was

received,  and you don't  think it is  valid,  just hit ESC.

That will  clear  the XOFF  flag and  allow you to  continue

work.

Carriage Return Translation

Use this  to set  the power  up default for  carriage return

translation. Some systems send only  a CR and expect  you to

provide the line feed, while others send both the CR and LF.

If this  parameter is  set to CR,  incoming carriage returns

are  left  alone.  If  set  to CR/LF,  any  carriage returns

received from the  remote will have a line feed  appended to

them by ProComm. Most TTY applications (including most BBSs)

send both the carriage return and line feed, so  the ProComm

default  is  CR.  You  can  change this  setting on the  fly

without affecting the default using the ALT-F3 command.

BS Translation

The   backspace   (BS)   keystoke   can    have    different

interpretations depending  upon  the  terminal type. In most

cases it is destructive, that is the  cursor will delete the

Copyright (C) 1986 PIL Software Systems                   15

ProComm 2.2                                 Program Features

character to the left when it moves there.  In  some  cases,

the  VT100 for instance,  the BS  behaves like a cursor left

command.  You  may  choose  the  correct  behavior for  your

application using  this parameter.  The  ProComm default  is

destructive.

Line Wrap

The line  wrap setting  controls  how lines greater than  80

characters are  handled. If  set on, those  lines will  wrap

around and be  displayed  on  the next line.  With line wrap

off,  lines  greater than  80 characters  in length will  be

truncated. The ProComm default is line wrap off.

Scroll

The  scroll  parameter  controls  what  happens  if  ProComm

receives  a CR (or CR/LF)  while the cursor is positioned in

the bottom line. If scroll is on, the screens scrolls 1 line

and  the  new  line  can be  printed.  If scroll is off, the

cursor  returns  to  the left most column, and the new  line

over prints the old. Normally this will  always  be  set on.

For  some full screen  applications you may need to  inhibit

screen scrolling.

Kermit Setup

------------

We  have  provided  control  over a number  of  Kermit  file

transfer parameters. This is not the place, however, to give

a Kermit  tutorial. If you  do  not know what these are, you

should   leave  them   as  they  are.  Consult   the  system

administrator  for your  remote  if you  have any  questions

regarding their Kermit installation.

A further  discussion of ProComm's Kermit  implementation is

available in the file transfer section.

General Setup

-------------

Editor Name

This parameter  can be set to the name of an  editor or list

program you wish to use from within ProComm. You may specify

it   as    a   complete    filename    with    path    (i.e.

C:\WP\EDIT\EDITOR.EXE),  just   the  program  name   without

extension (i.e.  EDITOR) or anything in  between. If no path

is specified, ProComm  will search the directories specified

Copyright (C) 1986 PIL Software Systems                   16

ProComm 2.2                                 Program Features

in the environment variable PATH for the indicated file. You

must have enough available RAM to load the desired program.

Default Log File

Use this  parameter to set the  default name for the file to

be used when file logging is  in effect. If the file exists,

new data will be appended to its end. If the file  does  not

exist it will be created in the current directory.

Default Screen Dump File

This option names the file to which  to append screen dumps.

If  it  does not  exist  when  ALT-G is pressed it  will  be

created in the current directory.

Screen Write Method

ProComm can use one  of two different methods to perform its

screen writing. Normally you would use DIRECT  screen writes

for speed.  In  some  cases,  such  as  under  multi-tasking

operating systems, you might wish to go through the BIOS for

screen writing. You may also select BIOS mode using the '/M'

command line option.

Translate Table

This  setting  controls  whether  or  not   translate  table

conversion is performed. Selecting YES causes  the translate

table to be  activated immediately,  as well as  on  program

startup. The translate  table  may be  defined  and  toggled

on/off using the ALT-W command as well.

Sound Effects

The  sound  effects  option   controls  the   existence   or

suppression of ProComm sound effects. This  setting controls

the window opening  and  closing noises primarily,  and does

not effect the alarm, the modem or  bells (^G) sent from the

host.

Alarm Sound

This setting  controls  whether or not the alarm  uses sound

effects. The  alarm is used  to  indicate  the  end of  file

transfers, connects during  re-dials  and  by the %A  script

command. If alarm sound is  ON, these events will trigger an

alarm clock like ringing  which will continue for the number

of seconds specified by the alarm time below. If alarm sound

is OFF, the appropriate message will flash for the indicated

time but will be silent.

Copyright (C) 1986 PIL Software Systems                   17

ProComm 2.2                                 Program Features

Alarm Time

Set this to the time (in seconds) that you wish the alarm to

sound. For instance, if you desire to have the file transfer

alarm ring for two minutes, set this to 120.

Exploding Windows

Beginning  with  version  2.2 you  may  select  what type of

windows  you wish ProComm to use. By  selecting YES for this

option you instruct ProComm to use  exploding windows, as it

did in previous versions.  Selecting  NO  causes windows  to

just appear without the expanding frames.

XMODEM Mode

Some remote  systems,  such as CompuServe, cannot handle the

normal  XMODEM  error timings. For  these  systems,  set the

XMODEM mode to RELAXED to avoid file transfer aborts due  to

timing errors.  In  most cases  this  should be  be  set  to

NORMAL.

ASCII Upload Pacing

ProComm performs line  pacing during ASCII uploads if you so

desire.  After  each line is sent  the program will wait for

the time specified here (in 1/10 of  seconds) before sending

the next  line.  Pacing may be  set to zero if  the host can

handle it. The default is 8 (.8 seconds).

ASCII Upload Local Echo

Use this setting  to  control whether or not  ProComm echoes

locally what is being  transferred during ASCII uploads.  In

most cases set this to NO and let  the host  due any echoing

desired.

ASCII Upload Strip LF

This option controls outgoing CR/LF translation during ASCII

upload.  If  set to  YES, line  feeds  are stripped from the

outgoing file. If  set to NO (the default), the file is sent

as is.

Translate Pause Character

Use  this parameter  to  set the  character which  is  to be

interpreted as a pause during output string translation. See

the section on output string translation for more details.

Translate CR Character

Use  this parameter to  set the character  which  is  to  be

interpreted  as  a  carriage  return  during  output  string

Copyright (C) 1986 PIL Software Systems                   18

ProComm 2.2                                 Program Features

translation.  See  the section on  output string translation

for more details.

Translate CTRL Character

Use this  parameter to set  the  character which  is  to  be

interpreted as  signaling a  control character during output

string  translation.  See  the   section  on  output  string

translation for more details.

Translate ESC Character

Use  this  parameter to set  the  character  which  is to be

interpreted as an  ESC during output string translation. See

the section on output string translation for more details.

Aborted Download

Use this setting to determine the disposition of files  that

are aborted during  downloads. If set to KEEP, these partial

files are kept on the  disk  and are your responsibility. If

set  to  DISCARD, they are  erased from  your disk  when the

download aborts.

DIALING DIRECTORY

-----------------

The ProComm dialing directory automates most of your dialing

tasks. The directory  holds information for 100 of your most

often called  systems. The information includes the name and

phone number, as well the communications parameters specific

to  that entry. Also  included  in  the  directory is a user

definable modem command and up to  four long  distance codes

that you can use with alternate long-distance services.

Press  ALT-D to  access  the dialing  directory. A window is

opened  that  displays pages of  10 entries  at a time.  The

current  modem command is displayed at the  bottom left, and

the  active long distance codes  are indicated to the right.

Long distance  codes are identified  by the characters  '+',

'-', '@' and '#'. The modem command defaults to 'ATDT'.

Use the PGUP and PGDN keys  to display the previous  or next

page of entries. The  up and down arrow keys will scroll the

display one entry in  either direction. The  HOME  key  will

position  the  display at the first page in  the  directory,

while the END key will  position you at the last page. Press

ESC to exit the dialing directory.

Copyright (C) 1986 PIL Software Systems                   19

ProComm 2.2                                 Program Features

Searching for an Entry

----------------------

Version  2.2  adds  a   search  capability  to  the  dialing

directory. To look  for a particular entry press 'F' or '/'.

A window will open asking for the text  to search for. Enter

the string you wish to  be found and press CR. If the string

is  found the dialing  directory will  scroll  to  the  page

containing that entry, and the entry will be highlighted.

Searching in the dialing directory is  not  case  sensitive.

For instance a search string  of 'abcd'  will  match 'abcd',

'ABCD', 'AbCd' or  any  other mix  of upper  and lower  case

characters.  If the search  routine reaches  the end  of the

dialing  directory without  finding  a match  it  will  wrap

around to the beginning  of the directory  and continue  its

search until it has checked every entry once.  If the string

is  still  not found a message  is  displayed.  To repeat  a

search for the same text, press 'F' or '/' a second time and

then immediately press CR.

Revising the Dialing Directory

------------------------------

When you first use the dialing directory most of the entries

will be cleared  to the default. The  name and phone  number

fields will be filled with periods; the default baud rate is

1200, with no parity, 8 data bits,  1 stop  bit and echo off

(full duplex). To  add  an  entry to  the  directory,  or to

change an existing entry,  enter 'R' at the  '==>' prompt. A

window will appear asking for the entry to revise. To revise

an entry type  the number of the entry to revise (1-100). To

revise  the  modem  command  enter  'M';  to revise  a  long

distance  code  enter  the  long  distance  code  identifier

(-,+,@,#).

Adding or Revising an Entry

If you chose to revise an entry the  current values for that

entry  are displayed.  You are  prompted first for the name.

Enter up to  24  characters. You can  use the  backspace and

left arrow keys to  edit  your entry. Press CR when  you are

done. Pressing CR without making  any changes will leave the

current  name intact. You will now be prompted for the phone

number. Enter only  the numbers you need; the number will be

right  justified on the page. Press CR when you are finished

entering the number. The current baud rate is now displayed.

Press any key  (except ESC) to rotate through the  available

baud  rates. Press  CR to select  the  one you want. Use the

same  technique  to select the parity, data bits, stop  bits

Copyright (C) 1986 PIL Software Systems                   20

ProComm 2.2                                 Program Features

and  echo.  After  specifying  the echo  you  will be  asked

whether to save the entry to  disk.  Enter 'Y' to  save your

changes, 'N' to abandon. You may also  abandon a revision at

any  point by pressing  ESC.  After  saving an  entry  it is

written to disk  and  the current page redisplayed. An entry

need not be displayed to be revised.

Revising the Modem Command

If you specified 'M' to revise the modem command the current

command   is  displayed.  Enter  a   new  command  that   is

appropriate for your modem. You may imbed pauses,  CRs, ESCs

or  control  characters  in  the  modem  string   using  the

procedure  described  in   the  section  on  output   string

translation. Press RETURN when you are done entering the new

command.  Pressing  RETURN  as  the first  character of  the

string will clear the command to blanks.

Now indicate  if the new command should be saved to disk. If

you enter 'Y' the new command is written to disk and will be

used  in future  executions of ProComm. If you say  'N'  the

changes will remain for the current session only. To abandon

the  changes  completely  press ESC  any  time  previous  to

answering the save prompt.

Any  changes  you  make are  immediately  reflected  in  the

window.  You may also revise the dialing command through the

setup facility described earlier.

Adding or Revising Long Distance Codes

Long distance  codes are  mainly used  to  access  alternate

long-distance  services.  Four  long   distance  codes   are

provided; they  are identified  by the characters '-',  '+',

'@' and '#'.  Press 'P' at the dialing directory prompt '==>

' to display the  current long distance codes. Press any key

to remove the long distance code window.

To revise a long distance code, enter the long distance code

identifier  when  prompted  for  the  entry  to  revise. The

current string for that long distance code  is displayed and

you  are  asked for  the new  entry. Control  characters and

carriage returns are allowed in long distance codes by using

the same conventions as in the modem command.  Press  RETURN

to complete the long distance code revision. Pressing RETURN

as the first character in the  new string deletes  that long

distance code. As with the modem command, if you specify not

to save the changes they will be  in effect for the  current

session  only.  Press  ESC  to  abandon  long distance  code

changes completely.

Copyright (C) 1986 PIL Software Systems                   21

ProComm 2.2                                 Program Features

Deleting Entries

----------------

You  can use  the 'D'  command to  delete entries  from your

dialing directory.  A  window  will  prompt  for a range  of

entries to delete. Leave the second field  blank to delete a

single entry. You'll be  asked to verify that  you want  the

entries deleted before any action is taken.

Making a Call

-------------

To dial  an entry in your dialing directory, simply type the

entry's  number at  the '==>  ' prompt. To dial with  a long

distance code, place  the  code's  identifier  before and/or

after  the  number. For  example, enter '14'  to dial  entry

number  14. Enter '#37' to dial entry number  37 preceded by

the long  distance  code identified by  '#'.  Enter '34+' to

dial entry number 34 followed by the '+' long distance code.

ProComm  will  send  the  modem  command,  an  optional long

distance  prefix,  then  the  number  and  an optional  long

distance suffix to the  modem. All  four  of  parts  of  the

dialing   string   are   translated  using  the  conventions

described  under output string  translation.  The parameters

for the dialed entry become  the current settings and remain

after the call is complete.

To dial a  number  not  in your  directory  use  the  manual

dialing  command  ('M'). Then  enter  the number you wish to

dial. ProComm will send the dialing command  plus the string

you entered. A long distance  code indicator may  be used if

it is the first or last character in the string.

Printing the Directory

----------------------

ProComm lets you  output  your directory to a printer,  disk

file or any  DOS device.  Type 'L'  at the '==> ' prompt and

then specify where to send the file.  Simply press RETURN to

use the  default  of  PRN  (LPT1:). You can even  output the

directory to the COM port you are using to send a listing of

your directory to the remote computer.

AUTOMATIC REDIAL WITH CIRCULAR DIALING QUEUE

--------------------------------------------

Some remote  systems  can be  very  hard to  reach.  ProComm

Copyright (C) 1986 PIL Software Systems                   22

ProComm 2.2                                 Program Features

facilitates reaching them with its automatic redial feature.

Press ALT-R to invoke the redial  procedure. Now specify one

or  more  dialing directory  entries  separated  by  blanks,

commas  or semicolons.  Entries  may  contain long  distance

codes.  The program will continuously  redial the numbers in

the list  until  you are  connected. If the  redial time  is

exceeded,  or ProComm senses  one of the  modem's no connect

strings  (specified in the  setup screen), the  program will

proceed to dial the next number in the list. As  each number

is reached, it is removed from the circular queue.

Pressing CR  without specifying  any  entries will  dial the

numbers that were in the list the last time it was used.

You can  set  the  amount  of  time ProComm waits  for  some

response by  pressing  the END key. Specify the  new time to

wait  (5-100 seconds)  and  press  RETURN. If  you  want the

change to become permanent respond 'Y' to the 'save to disk'

prompt. You can also specify this timeout delay in the setup

screen.

Between each call ProComm will delay for the amount of  time

specified by the modem pause delay parameter (setup screen).

This is provided  to allow  some modems a sufficient time to

reset  between calls. To continue with  the next call  press

any key (except ESC).

The alarm will sound when you connect during a redial.

FILE TRANSFER

-------------

The ability  to transfer information makes  a communications

program  a  very powerful  tool.  ProComm  gives  you  seven

choices for  two-way information transfer  via file transfer

protocols.  With  them you  can  up- and download files from

almost any  system. The different  protocols  are  described

below.

Uploading Files

---------------

Begin  uploading a  file to  another system by starting  the

transfer on  the remote  computer. When the remote indicates

it is ready press PgUp. You will be presented with a menu of

protocol choices.  Enter the number of the protocol you wish

to use. A second window will prompt you for the name of  the

file to upload.  Enter the filename, including a path if you

want,  and press return.  Another  window  will  open  which

reports on the progress of the transfer.

Copyright (C) 1986 PIL Software Systems                   23

ProComm 2.2                                 Program Features

An  ASCII upload is somewhat  of a special case. No transfer

progress window will  be displayed but the status  line will

indicate that  an  ASCII transfer  is  taking  place.  ASCII

uploads are under  the control of 3 setup options. The first

option controls line pacing. You can set up ProComm to pause

after  it sends each line of text to  avoid  overflowing the

receiver.  You can also  specify  if  text is  to be  echoed

locally  and  if  linefeeds  should  be  stripped  from  the

outgoing file.

Downloading Files

-----------------

Downloading  files  is  just as  easy.  After  starting  the

download procedure  on the remote press  PgDn. The  protocol

selection  window  is  displayed. Enter the  number  of  the

protocol to use.

If you select  ASCII, XMODEM or YMODEM protocol you will  be

prompted for the filename.  In all other cases  the filename

is  provided by the sender. If you enter the  name of a file

that already  exists you  will  be  asked  whether or not to

overwrite it.  If a filename provided by the sender  already

exists the received file will be renamed by placing a dollar

sign in the first position of  the filename. For example, if

you  select to  download FILE1.EXE using the Telink protocol

and  that file already  exists in the current directory, the

downloaded file will be named $ILE1.EXE

After specifying the protocol and (possibly) the file  name,

a  window  will  open  which  displays  various  information

regarding the transfer. When the transfer is complete, or an

abort  is  sensed, the  alarm  will  sound  and  appropriate

messages will be displayed.

ASCII downloads  are  somewhat different from  the others in

two respects. First, no transfer status window is displayed.

A  message  on  the  status  line indicates  that  an  ASCII

download is in  affect.  Second, ASCII downloads  need  user

intervention to  complete. Text is captured until  you press

ESC to terminate the transfer.

File Transfer Protocols

-----------------------

ASCII

ASCII file transfer is the equivalent of typing  information

from one system to another. The ASCII characters are sent in

Copyright (C) 1986 PIL Software Systems                   24

ProComm 2.2                                 Program Features

a one-way stream with no handshaking (other  than XON/OFF if

activated) or error checking performed. This method is  fine

for some  applications, but  you  are limited  to  text file

transfer.

One difference between an ASCII download and file logging is

that  all data  (including  terminal  escape  sequences)  is

captured, whereas during logging terminal control  sequences

are filtered out (with the exception of formfeeds).

XMODEM File Transfer

XMODEM is a block-oriented error checking  protocol released

into the public domain  by its creator Ward  Christensen. It

is very popular on electronic bulletin board systems. XMODEM

transfers  only  a single  file at a time. The protocol uses

two-way  communications  and  either  a checksum  or  cyclic

redundancy check for error checking. XMODEM can handle  text

or executable files with over 99% accuracy. ProComm supports

and  automatically  adjusts  for both the checksum  and  CRC

variants.

The  XMODEM protocol is defined  such  that  CRC checking is

always  attempted first.  If  CRC is not acknowledged by the

sender then the  checksum method is  used. ProComm correctly

implements  this,  however some  other  systems  do  not.  A

problem  can arise if the remote system  responds to the CRC

attempt but uses checksums.

Note that XMODEM requires transfers  to be performed  with 8

data bits, 1 stop bit and no parity. If you attempt to begin

an XMODEM transfer while set to  other  parameters,  ProComm

will  automatically switch  to N/8/1, returning you to  your

original parameters when the transfer is complete.

Some systems,  such  as CompuServe, have trouble meeting the

standard  XMODEM timing  sequences. You  may  set  ProComm's

XMODEM  facility  into  a  'relaxed'  mode,  which  has less

critical  timing,  using  the  setup  (ALT-S)  screen.  Most

systems will work fine with XMODEM set to normal.

MODEM7 File Transfers

MODEM7 is a variant of the XMODEM protocol. By  sending  the

filename   batch   transfers   (multiple   files)   can   be

accomplished. CRC and checksum are supported.

YMODEM File Transfers

YMODEM  is  another  XMODEM variant.  Its  main advantage is

support for longer  data blocks  to  speed  transfer  times.

ProComm supports YMODEM for single file transfers and YMODEM

Batch  for  multiple  file  transfers.  YMODEM   Batch  also

provides  some  header  information  such  as  filename  and

Copyright (C) 1986 PIL Software Systems                   25

ProComm 2.2                                 Program Features

filesize.  YMODEM file transfers are  always  performed with

CRC error checking.

Telink File Transfers

Telink is yet another XMODEM/MODEM7 variant which  adds file

size  and creation date information. It is found  mainly  on

FIDO bulletin  board systems, and  provides for  batch  file

transfers.

Kermit File Transfer

Kermit is  a  packet-oriented protocol developed at Columbia

University that  is  available  on  many  different computer

systems. By using a technique called 8th-bit quoting, Kermit

is able  to  transfer  binary  files  between  7 and  8  bit

systems.  In  some implementations, such as ProComm,  Kermit

supports multiple file transfers.

Our  implementation  of Kermit  includes all  of  the latest

kermit   enhancements,   such  as  data   compression,  file

attributes, and sliding windows.

The most significant of these features is sliding windows. A

"sliding window" protocol is a full duplex protocol that can

transmit  and receive  data  at  the same time.   The XMODEM

family  of protocols are  half duplex protocols.  They  must

wait between each  block of data for a reply from  the other

side. This wastes  a lot of time. Full duplex  protocols can

send a continuious stream of data while  receiving replys at

the  same  time.   This  greatly  increases  file   transfer

effeciency. ProComm Kermit will  automatically sense if  the

other Kermit  supports  sliding windows and will use them if

it does. Currently Sliding Window Kermit is available on The

Source, TCOMM BBS,  and  PC-HOST BBS. Mainframe versions are

under  developement and  should  be  appearing soon. ProComm

Kermit  is  backwards  compatible  with earlier versions  of

Kermit.

ProComm's default kermit  settings  are fairly  standard and

should only need  to  be  changed under special  conditions.

Due to  the  sliding window enhancment, block size should be

limited to a maximum of 90,  rather  than 94 as  in standard

Kermit.

In addition,  a few  Kermit server  commands are  supported.

Press  ALT-K  to  access  the  Kermit server  command  menu.

Available commands include  Finish  and Logout.  Finish  (1)

will  terminate the Kermit  session  and return  you to  the

system level. Logout (2) will terminate Kermit and  log  you

off the remote.

More  information  on  Kermit  is  available  from  Columbia

University. Send $5 each for the Protocol or User manual to:

Copyright (C) 1986 PIL Software Systems                   26

ProComm 2.2                                 Program Features

Kermit Distribution

Columbia University Center for Computing Activities

7th floor, Watson Laboratories

612 West 115th Street

New York, NY 10025

TRANSLATE TABLE

--------------

ProComm's  translate table provides a means for you to strip

or  replace  characters you receive  from the  remote. Press

ALT-W to display the current translation settings. The table

will indicate the whether  translation  is  currently taking

place, as well as showing you the current translation values

for   the  ASCII  characters  0-127.  Press  F2  to  display

translation values for characters 128-255. Pressing the keys

F3 and F4 will toggle the translation effect  on or off. You

may set the default condition using the setup screen.

To change a translation  value,  enter  the  ASCII  code  to

reset. Now enter the translated value (both  in decimal). To

strip an unwanted character, translate it to 0. Press  F1 to

save  translation  values  between  sessions.  Press ESC  to

return to terminal mode.

COMMAND FILES

-------------

Command files are text files you create that contain ProComm

commands.  You can  use  command files to  perform automatic

logons, do unattended file transfers and  many  other tasks.

Create a command file using  your favorite  word  processor.

You may use any valid filename; however,  ProComm  looks for

files  with  an extension  of .CMD  when it lists  available

command files. Use the commands described below  to  perform

your specific  task.  Be  sure  to test  your command  files

thoroughly before using them for unattended communications.

You can  execute command files from  within ProComm  or as a

command  line option when  you first invoke  the program. If

you specify them on the command line, using the '/F' option,

they will  be executed  immediately  upon  program  startup.

Execute them inside  the program by pressing ALT-F5. ProComm

first searches  for files  with the  .CMD  extension  in the

current directory.  If any  are found they  are displayed in

the window. If none are found ProComm looks in the directory

pointed to by the PROCOMM environment variable. Again, if it

finds any they are  displayed  in  the window, otherwise the

message 'NO  FILES' is  displayed. You may  then specify the

Copyright (C) 1986 PIL Software Systems                   27

ProComm 2.2                                 Program Features

command by  file  name, or  chose from the  scrolling window

display.

To chose from the scrolling window, position the highlighted

bar on  the name of the file  you  wish to execute and press

enter.  PGUP and  PGDN will  scroll  the window  one page in

either direction. The cursor keys will scroll the window one

entry  in  either  direction.  Pressing  the HOME  key  will

position  you at the first page of  available command files;

pressing END will position you at the last.

If you begin to enter a command file name at the  prompt you

may still scroll the  window  but you may not  select a file

for execution from the  scrolling display. You can leave the

.CMD  extension off  when executing command files explicitly

by name.

You can abort  a  command file  by  pressing  ESC during its

execution. The  command  file  will  abort when  the current

command  has completed (this might take  a  few seconds). In

some  cases,  such  as while  dialing  or performing  a file

transfer, two  ESCs  are needed: one  to abort  the task  in

progress and a second to abort the command file.

There is  a special command file named  PROFILE.CMD. ProComm

will look for this file in the default directory first,  and

then in the directory pointed to by the PROCOMM= environment

variable discussed  above.  If  it  is  found,  it  will  be

executed  immediately upon program startup. You can  use the

profile to  do custom setup work  like changing to a special

directory,  turning  off the sound,  changing  communication

parameters or overriding the  default  modem  initialization

string.

Command file instructions  all have the  same  general form.

Each  must  begin  with  a  percent  sign  ('%'),   followed

immediately by  a 1 to 3 character command. Commands  may be

in either upper or lower  case. All other characters  in the

file are ignored and may be used as comments.

Many commands also require an additional argument. There are

two conventions for specifying arguments. If the argument is

enclosed in  quote marks  (the  double  quote  char:"), that

argument is used. If the argument  is enclosed in tick marks

(the single quote char:'), a window is opened which uses the

specified argument to prompt the user for the real argument.

For  example, the  command  %XUK"FILE1.EXE"  causes the file

FILE1.EXE to be  uploaded via Kermit. The command  %XUK'FILE

TO UPLOAD' will open a window containing the prompt 'FILE TO

UPLOAD'.  The  user must then specify the file he  wishes to

have uploaded.  That filename  is then  passed to the Kermit

upload procedure and  execution  continues. Be very  careful

that you do not  specify arguments  to be prompted for in  a

command file that is to execute without user supervision.

Copyright (C) 1986 PIL Software Systems                   28

ProComm 2.2                                 Program Features

We  have provided several  command files  for  accessing the

popular  information  services.  Look  at  these  files  for

examples of correct command usage.

ProComm currently supports the following commands:

%A - Alarm

This command will sound an alarm to alert you to some event.

Set the  alarm time from the setup screen or  using  the %ST

command. This  command  is  also under control of the  alarm

on/off  setting  from   the  setup  screen,  which   may  be

controlled by the %SA command.

%B"directory" - Change default directory

This command will change the default  directory or drive. It

will  not do both at once however. For instance, the command

%B"A:DIR1" will make  DIR1  the  current directory for drive

a:, but it will  NOT  make ProComm change default  drives to

a:. Use a command like %B"A:" to change drives.

%C"ldcode number ldcode" - Call a number

Use  the  call  command to dial  an  entry  in your  dialing

directory.  Specify  the number  of  the  entry,  optionally

preceded and/or followed by a long distance code identifier,

as  the quoted  argument. For example.  the  command %C"#24"

will  dial  entry  number  24  using the  long distance code

identified  by  '#'.  This  command  uses  the  auto  redial

facility to place its calls; it will keep re-dialing until a

connection is made. Use the '%R' command described below  to

change the redial wait delay. Please note  that  if you sort

your dialing directory you  will need to change your command

files so the entry numbers match.

%D"command to execute" - DOS Command

This feature allows  you  to execute a DOS command  or other

program from the command file. Enter the command as it would

appear on  the DOS command  line  between  the  quotes.  For

example, to go  out to DOS  and type a file  to your printer

use  the  command  %D"type  filename  >  prn". This  command

requires  two  things: enough  memory  and  the  environment

variable COMSPEC=  set correctly. CAUTION: If you execute  a

program  or  command requiring  user input be  sure you  are

around to provide it, as the program will wait until you do.

%Ex - Emulations, where x is 0,1,2,3,4,5,6,7,8 or 9

Use this command to  specify which terminal emulation  is to

be used.

Use 0 for IBM 3101

Copyright (C) 1986 PIL Software Systems                   29

ProComm 2.2                                 Program Features

1 for DEC VT-100

2 for Televideo 910/920

3 for Televideo 925/950

4 for DEC VT-52

5 for Lear Siegler ADM3/5

6 for Heath/Zenith 19

7 for ANSI-BBS

8 for ADDS Viewpoint

and 9 for Wyse 100.

%F"x" - Change communications parameters

Changes the comm parameters based on 'x' as follows:

1 - 300/E/7/1

2 - 1200/E/7/1

3 - 2400/E/7/1

4 - 4800/E/7/1

5 - 9600/E/7/1

6 - 19200/E/7/1

7 - 300/N/8/1

8 - 1200/N/8/1

9 - 2400/N/8/1

10 - 4800/N/8/1

11 - 9600/N/8/1

12 - 19200/N/8/1

13 - Odd parity

14 - Mark parity

15 - Space parity

16 - 7 data bits

17 - 8 data bits

18 - 1 stop bit

19 - 2 stop bits

20 - COM1:

21 - COM2:

Note that these settings correspond  to  those  used  by the

ALT-P command.

%G - Screen dump

%G  will  make a screen dump  of the current screen contents

and append it to the current .IMG file.

%H - Hang-up

This instruction is the same as the  ALT-H command described

below.

%I"string to wait for" - Incoming

This handy feature  will  pause command file execution until

the specified string is received from the remote system,  at

which point execution  will  continue.  Strings  must  match

Copyright (C) 1986 PIL Software Systems                   30

ProComm 2.2                                 Program Features

exactly  in all  respects including  case. To avoid deadlock

situations there is a maximum wait time for any  string. The

default wait time is 30 seconds but may be changed using the

'%P' command described below. If the  delay time is exceeded

without the string being found, execution will continue.

%Jx - ALT key sequence, where x is 0 to 9

Use this  to execute a  keyboard  macro.  For instance,  use

'%J1' to issue the sequence assigned to ALT-1.

%KBB - Kermit binary mode

Set Kermit into binary mode.

%KBT - Kermit text mode

Set Kermit into text mode.

%KF - Kermit server command: Finish

Shuts down the Kermit server and  returns  you to the system

level.

%KL - Kermit server command: Logout

Shuts down the Kermit server and logs you off.

%LO"filename" - Log file open

This instruction will open the log file using  the indicated

name.

%LC - Log file close

Close  the  previously opened  log  file  by  including this

command.

%LH - Log file hold

Temporarily suspend logging to the opened log file.

%LR - Log file resume

Resume logging to the log file.

%LPO - Open printer for logging.

Begin logging to the printer.

%LPC - Close printer.

Stop logging to the printer.

Copyright (C) 1986 PIL Software Systems                   31

ProComm 2.2                                 Program Features

%M"message" - Message

Display  a message on your screen. This message  will not be

sent to the remote system.

%NY"prompt" - Get and transmit user input

This command will open a window and prompt for user input to

send to the  remote. The window will accept strings up to 68

characters long. The prompt may be up to 60  characters. The

string may contain control  characters as  described  in the

section  on output  string  translation. The string sent  is

followed by a carriage  return. This  command will not  time

out; do not use it for unattended operation.

%NN"prompt" - Get and transmit with mask

This  command works the same  as the  %NY command  with  one

exception: text entered  by the user at the prompt is masked

by asterisks for security purposes.

%OD - Direct screen writes

Use the direct screen write method.

%OB - BIOS screen writes

Use the BIOS for screen writes.

%P"delay in seconds" - Pause for incoming delay

Use  this  command  to change  the amount  of time the  '%I'

command  waits  for  an  incoming  string.  If  you  specify

'%P"45"', any  subsequent  '%I' command will pause up to  45

seconds waiting for the string.

%Q - Quit

End the current ProComm session.

%R"delay in seconds" - Redial delay time

Use this  to  reset the amount of  time  allocated for  each

redial  attempt. If  the  number  of  seconds  specified  is

reached  before a  connect  or no connect  message  has been

received from  the modem, ProComm will hang up the call  and

attempt another redial.

%SAN - Alarm off

This command will cause the alarm sound  to be supressed. It

does not affect window sound effects,  the modem speaker nor

bells (^G) received from the remote.

Copyright (C) 1986 PIL Software Systems                   32

ProComm 2.2                                 Program Features

%SAY - Alarm on

This  command will cause the alarm sound to be activated. It

does  not affect window sound effects, the modem speaker nor

bells (^G) received from the remote.

%SN - Sound off

Turn  sound  effects off. This  does  not affect  the  modem

speaker, the alarm or bells (^G) sent by the remote.

%SY - Sound on

Turn  sound effects  on.  This  does  not  affect the  modem

speaker, the alarm or bells (^G) sent by the remote.

%ST"alarm time in secs" - Set alarm time

Set the amount of time the alarm will sound.

%T"string" - Transmit string

Use  this  command  to  transmit  a  string  to  the  remote

computer. You may imbed CTRL  characters  by prefacing  them

with the translate CTRL character (see the section on output

string translation).

%U1 - Clear screen

Clears the local console only.

%U2 - CR

Incoming carriage returns are displayed as is.

%U3 - CR/LF

Incoming  carriage  returns  are  displayed  as  a  carriage

return-line feed sequence.

%U4 - Full duplex

Set duplex to full (no echo).

%U5 - Half duplex

Set duplex to half (echo performed).

%U6 - Break

This command causes the break condition to occur.

Copyright (C) 1986 PIL Software Systems                   33

ProComm 2.2                                 Program Features

%U7 - Set line wrap on

Cause lines  over 80 characters to wrap around  to  the next

line.

%U8 - Set line wrap off

Cause lines longer than 80 characters to be truncated.

%U9 - Set scroll on

Set scroll on. See the section on setup for more details.

%U0 - Set scroll off

Set scroll off.

%UBN - BS destructive

Sets backspace translation to be destructive.

%UBN - BS non-destructive

Sets backspace translation to non-destructive.

%UHN - Handshake none

Tells ProComm not to perform any handshaking protocol.

%UHY - Handshake XON/XOFF

Instructs ProComm to use XON/XOFF handshaking.

%UXN - XMODEM normal

Set XMODEM to use the normal mode.

%UXR - XMODEM relaxed

Use relaxed XMODEM for file transfers.

%W"wait time in seconds" - Wait

This provides a pause of the given duration.

%XDK - Download Kermit

Download file(s) using the Kermit protocol.

%XDM - Download MODEM7

Download file(s) using the MODEM7 protocol.

Copyright (C) 1986 PIL Software Systems                   34

ProComm 2.2                                 Program Features

%XDT - Download Telink

Download file(s) using Telink.

%XDX"filespec" - Download XMODEM

Download using XMODEM to filespec.

%XDY"filespec" - Download YMODEM

Download using YMODEM to filespec.

%XDY - Download YMODEM batch

Download file(s) using YMODEM batch protocol.

%XUA"filespec" - Upload ASCII

Upload the file called 'filespec' using ASCII protocol.

%XUK"filespec" - Upload Kermit

Upload the file called 'filespec' using Kermit protocol.

%XUM"filespec" - Upload MODEM7

Upload the file called 'filespec' using MODEM7 protocol.

%XUT"filespec" - Upload Telink

Upload the file called 'filespec' using Telink protocol.

%XUX"filespec" - Upload XMODEM

Upload the file called 'filespec' using XMODEM protocol.

%XUY"filespec" - Upload YMODEM

Upload the file called 'filespec' using YMODEM protocol.

%XUZ"filespec" - Upload YMODEM batch

Upload  the   file  called  'filespec'  using  YMODEM  batch

protocol.

%YEN - ASCII upload echo off

Do not echo text locally during ASCII uploads.

%YEY - ASCII upload echo on

Echo text locally during ASCII uploads.

Copyright (C) 1986 PIL Software Systems                   35

ProComm 2.2                                 Program Features

%YP"pace count in seconds" - ASCII upload pacing

Change the pace count for ASCII uploads.

%YSN - ASCII upload strip LF off

Do not strip LFs when uploading via ASCII file transfer.

%YSY - ASCII upload strip LF on

Strip LFs when uploading text via ASCII file transfer.

%ZY - Translate table on

Set incoming character translation active.

%ZN - Translate table off

Disable the translate table.

KEYBOARD MACROS

---------------

Another  useful  feature  provided  by  ProComm is  keyboard

macros. This facility allows you to assign character strings

to each of  the keys ALT-0 through ALT-9. The strings may be

up  to  24 characters in  length,  and  may contain imbedded

control  codes  and carriage  returns  using  the  translate

conventions described elsewhere. To  send the string  you've

assigned, simply press the appropriate key.

Access the keyboard macros  by pressing ALT-M  from terminal

mode.  A   window  will  appear   listing  the  current  key

assignments. To revise an assignment press 'R', and then the

key to assign (ALT-0 through ALT-9). Now type the string you

wish assigned to that key. Respond 'Y' to the save prompt to

make the assignment permanent; otherwise,  it  is discarded.

Press ESC to return to terminal mode.

DOS GATEWAY

-----------

ProComm provides a gateway to  DOS to allow you  to  execute

DOS  commands or other  programs  without ending the ProComm

session  - just  press  ALT-F4.  ProComm  uses  the  COMSPEC

environment variable to determine  the name of  the  command

processor to use, so that variable must be set correctly for

the gateway  to work. Enter  'EXIT'  on the DOS command line

when you wish to return to ProComm. CAUTION: executing other

communications  programs  through   the  gateway  may  cause

Copyright (C) 1986 PIL Software Systems                   36

ProComm 2.2                                 Program Features

erratic results when you return to ProComm.

EDITOR

------

To call  an editor, word  processor or  other  program  from

within ProComm,  press  ALT-A. ProComm will  attempt to find

and load  the  program named in  setup  screen. This  can be

quite useful  for  viewing  a file,  doing some  editing  or

whatever. The conditions described above for the DOS gateway

must  be  met  for this  feature  to perform  correctly.  In

addition, the  program  to be  executed  must  reside in the

current directory or in a directory  that is included in the

PATH.

There is no default for this feature so be sure to set it up

before attempting to use it.

SCREEN DUMP

-----------

Pressing ALT-G produces a screen dump  of the current screen

contents. Screen contents are  appended  to the current .IMG

file, which  may be specified  using the setup  screen.  The

default screen dump file is PROCOMM.IMG.

FILE AND PRINTER LOGGING

------------------------

ProComm  provides  2  alternatives  to  file  transfer   for

capturing  information:  file  and  printer  logging.  After

initiating  the logging  procedure, any  information  coming

from  the remote system is routed  to the log as well as the

screen.

Press ALT-L to  begin capturing information to your printer.

A message on  the status line will indicate that the printer

log  is open. ProComm writes  the  logged  data to  the  DOS

device PRN. You  may use  the DOS 'MODE' command to redirect

PRN. Press ALT-L again to toggle printer logging off.

Data can be captured  to disk  by using the  ALT-F1 command.

Specify the name of the file to be used,  or press RETURN to

use the default. If the file already exists, new information

will be appended  at the end of the  file.  Press  ALT-F2 to

suspend information  capture without closing the log file. A

message  will  be displayed on the bottom line of the screen

indicating logging status.

Copyright (C) 1986 PIL Software Systems                   37

ProComm 2.2                                 Program Features

Please note that terminal control sequences are not included

in the log files, with the exception of formfeeds.

SETTING DISPLAY COLORS

----------------------

To  set colors enter ALT-Z from terminal mode. A window will

open  with  a  list  of window selections  on the  left  and

instructions on  the right. Press the up  or down arrow keys

to choose a  selection to  change.  Next, press the  left or

right   arrow  keys  to   choose  which   colors  to  change

(background, foreground, or hi-lighted). Pressing the  space

bar will cycle  between the possible colors. After you  have

set all the colors you desire,  press ESC to  exit. You will

then be asked if you want to save the color changes to disk.

If you save the changes to  disk, they will remain until you

change them  again, otherwise they  will remain only for the

current session.

Some windows  do  not  appear  in  the window  list  but are

settable anyway. The ALT-T, ALT-B and ALT-X windows use  the

same  colors  as  the  keyboard  macros  window. The  redial

display  uses the phone directory colors.  The setup screens

use the help window colors. Communication screen  colors are

used  in  the  redisplay section. The  script file selection

window uses the file transfer window colors.

REDISPLAY BUFFER

----------------

To redisplay lines that have scrolled off your screen, press

ALT-F6.  ProComm will display the last  4000 characters that

have come in.

Use the PgUp  and PgDn  keys to scroll  one page  in  either

direction. Use the  up and down  arrow keys to move one line

in either direction. Pressing  the Home key will take you to

the first page of  the redisplay buffer, while  pressing the

End key will  take you to the  last. To  search for specific

text  press 'F' or '/'. You will be prompted for the text to

search  for. If the text if found ProComm will scroll to the

page it is on and highlight what it found. To search for the

same text again use  the same keystroke and then press ENTER

when  prompted for the text to  look  for. Searches are  not

case sensitive.

Copyright (C) 1986 PIL Software Systems                   38

ProComm 2.2                                 Program Features

FILE DIRECTORY

--------------

Press ALT-F  to get a directory listing of your files. Enter

a filespec  at the prompt just  as you would in  the DOS DIR

command. Paths and drives are supported. COMSPEC must be set

correctly for this feature to work.

VIEW A FILE

-----------

To  examine a file that  exists  on your system  type ALT-V.

You'll  be prompted  for  the name  of  the file.  Enter the

complete  filename,  including drive and path  designator if

desired. Press any key to begin viewing; press the space bar

anytime during viewing  to pause the display. Press  ESC  to

abort the viewing procedure.

PROGRAM INFORMATION

-------------------

To  display  the  ProComm program information screen,  press

ALT-I. Type any key to return.

CHANGE LOGGED DRIVE OR DIRECTORY

--------------------------------

ProComm  allows  you to change  the default directory or the

logged  drive you  are working in. Just press ALT-B and give

the name of the new directory or drive. All downloaded files

are  placed in the current default directory, as well as log

files you create.

DISPLAY ELAPSED TIME

--------------------

Pressing  ALT-T  will display the current time and date,  as

well  as the elapsed time  since  the last  call  was  made.

Elapsed time is reset  every time you make  a call using the

dialing directory, and when you make a  connection using the

redial facility.

Copyright (C) 1986 PIL Software Systems                   39

ProComm 2.2                                 Program Features

CLEAR SCREEN

------------

Press ALT-C  to clear your screen and home the cursor.  This

is a local effect only.

HANG-UP

-------

Press ALT-H  to  hang-up your  telephone connection. ProComm

accomplishes this by first attempting to  drop DTR.  If this

fails, then  the modem hang-up string is sent  to the modem.

ProComm  uses the  "carrier detect mask" in the setup screen

to determine if the hang-up was successful.  Most modems use

a  mask of 32  or 128.  If you receive the  "HANG UP FAILED"

message  after pressing ALT-H. perform the following checks:

Be sure your modem is set to monitor DTR and  CD. If hang-up

still fails,  check that the  modem  hang-up string  in  the

setup screen is correct for  your modem.  If it still fails,

try  different  values  for  the carrier  detect mask.  Some

common values are 16, 32, 48, and 128.

TOGGLE DUPLEX

-------------

ALT-E  will toggle duplex  between  full and half. ProComm's

default duplex  is set by the user in  the setup screen.  If

characters  you  type  appear twice you should set duplex to

full.  Likewise, if you type something that is not displayed

try toggling duplex to half. A message is  briefly displayed

on the status line when you toggle this setting.

TOGGLE CR - CR/LF

-----------------

Some  remote  systems  delimit lines  of  text  with  single

carriage returns, while others  use a  carriage  return/line

feed  combination.  ProComm  can add  a line  feed  to  each

incoming carriage return if  needed.  Use  ALT-F3 to  toggle

this  on  and off. The default  is CR only.  If  text  lines

overwrite each other, the  remote  is  not  sending  LFs, so

switch  to CR/LF. If text appears  double spaced  use the CR

setting. A message is briefly displayed when you toggle this

option.

Copyright (C) 1986 PIL Software Systems                   40

ProComm 2.2                                 Program Features

BREAK

-----

Pressing  CTRL-BREAK  will cause a  350  millisecond spacing

condition  (break)  on  the line.  Use  the  keyboard  macro

feature if you  require a specific character be sent for the

remote to realize a break.

ENDING A PROCOMM SESSION

------------------------

When your  communications  work  is  finished, press  ALT-X.

You'll be asked to  verify that you wish to leave. Enter 'N'

to continue working or 'Y' to exit.

Copyright (C) 1986 PIL Software Systems                   41

Appendix A                                Terminal Emulation

Overview

ProComm emulates  a  number of  popular  terminals.  ProComm

handles  most  of  the  standard features  and many  of  the

extended features  of the  terminals  emulated. Functions of

these terminals that are not emulated are still processed to

insure that all control codes are handled properly.

Due to the  wide variety of protocol convertors on mainframe

front-end   processors,   some  keys  may  not  function  as

expected.  These  emulations  have  been  tested   with  the

mainframe  systems  at the University  of Missouri, Columbia

and the  University  of California,  Berkeley,  as  well  as

within private industry.  If you encounter a problem,  refer

to the  TRANSMITTED CODES column in the  terminal  emulation

charts on the following  pages to see if ProComm  is sending

the code your protocol convertor is expecting.

If you encounter any errors in these emulations or have  any

questions, please let us know.

Copyright (C) 1986 PIL Software Systems                   42

Appendix A                                Terminal Emulation

Digital Equipment Corporation VT-100

ProComm supports the standard model VT-100 functions in ANSI

mode. Supported functions include: full/half duplex,  origin

mode, scroll  region, special graphics character set, keypad

application mode, cursor control,  erase functions, and full

display   attributes   (including   extensions    for   ANSI

color/graphics). 132 column mode  is not supported.  ProComm

responds  to the the Identify and Device Attributes commands

with ESC[?1;2c  (VT-100  with advanced video option). Keypad

Application Mode functions  are  mapped to ProComm  function

keys as shown in the chart below.

ProComm KEYS     DEC VT-100 FUNCTION       TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Delete ......... Character Delete ..................... 0x7F

Home ........... Home Cursor ...................... 0x1B5B48

Cursor Up ...... Cursor Up ........................ 0x1B5B41

Cursor Down .... Cursor Down ...................... 0x1B5B42

Cursor Left .... Cursor Left ...................... 0x1B5B44

Cursor Right ... Cursor Right ..................... 0x1B5B43

Ctrl-PgDn ...... Clear screen ............. 0x1B5B481B5B324A

End ............ Erase end of line ................ 0x1B5B4B

F1 ............. Keypad Application mode 1 ........ 0x1B4F71

F2 ............. Keypad Application mode 2 ........ 0x1B4F72

F3 ............. Keypad Application mode 3 ........ 0x1B4F73

F4 ............. Keypad Application mode 4 ........ 0x1B4F74

F5 ............. Keypad Application mode 5 ........ 0x1B4F75

F6 ............. Keypad Application mode 6 ........ 0x1B4F76

F7 ............. Keypad Application mode 7 ........ 0x1B4F77

F8 ............. Keypad Application mode 8 ........ 0x1B4F78

F9 ............. Keypad Application mode 9 ........ 0x1B4F79

F10 ............ Keypad Application mode 0 ........ 0x1B4F70

Shift-F1 ....... Program Function 1 (PF1) ......... 0x1B4F50

Shift-F2 ....... Program Function 2 (PF2) ......... 0x1B4F51

Shift-F3 ....... Program Function 3 (PF3) ......... 0x1B4F52

Shift-F4 ....... Program Function 4 (PF4) ......... 0x1B4F53

Shift-F5 ....... Keypad Application mode DASH ..... 0x1B4F6D

Shift-F6 ....... Keypad Application mode COMMA .... 0x1B4F6C

Shift-F7 ....... Keypad Application mode PERIOD ... 0x1B4F6E

Shift-F8 ....... Keypad Application mode ENTER .... 0x1B4F4D

Copyright (C) 1986 PIL Software Systems                   43

Appendix A                                Terminal Emulation

Mapping of VT-100 keypad application mode functions

PROCOMM FUNCTION KEYS              DEC VT-100 KEYPAD

F1 - F10      SF1 - SF10

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

| 1  | | 2  |  |PF1 | |PF2 |     |PF1 | |PF2 | |PF3 | |PF4 |

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

| 3  | | 4  |  |PF3 | |PF4 |     | 7  | | 8  | | 9  | | -  |

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

| 5  | | 6  |  | -  | | ,  |     | 4  | | 5  | | 6  | | ,  |

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

+----+ +----+  +----+ +----+     +----+ +----+ +----+ +----+

| 7  | | 8  |  | .  | |ENTR|     | 1  | | 2  | | 3  | | E  |

+----+ +----+  +----+ +----+     +----+ +----+ +----+ | N  |

+----+ +----+  +----+ +----+     +-----------+ +----+ | T  |

| 9  | | 0  |  |    | |    |     |     0     | | .  | | R  |

+----+ +----+  +----+ +----+     +-----------+ +----+ +----+

Copyright (C) 1986 PIL Software Systems                   44

Appendix A                                Terminal Emulation

IBM 3101

ProComm  supports   the  standard  IBM   3101  Model   1x/2x

functions.   Block  mode  and  transparent  mode   are   not

supported. Supported  functions  include:  full/half duplex,

full  character  set, scroll on/off, program function  keys,

cursor control, and erase functions.

ProComm KEYS     IBM 3101 FUNCTION         TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Delete ......... Character Delete ..................... 0x7F

Home ........... Home Cursor ........................ 0x1B48

Cursor Up ...... Cursor Up .......................... 0x1B41

Cursor Down .... Cursor Down ........................ 0x1B42

Cursor Left .... Cursor Left ........................ 0x1B44

Cursor Right ... Cursor Right ....................... 0x1B43

Ctrl-PgDn ...... Clear screen ....................... 0x1B4C

End ............ Erase end of line .................. 0x1B49

Ctrl-End ....... Erase end of screen ................ 0x1B4A

F1 ............. Program Function 1 (PF1) ......... 0x1B610D

F2 ............. Program Function 2 (PF2) ......... 0x1B620D

F3 ............. Program Function 3 (PF3) ......... 0x1B630D

F4 ............. Program Function 4 (PF4) ......... 0x1B640D

F5 ............. Program Function 5 (PF5) ......... 0x1B650D

F6 ............. Program Function 6 (PF6) ......... 0x1B660D

F7 ............. Program Function 7 (PF7) ......... 0x1B670D

F8 ............. Program Function 8 (PF8) ......... 0x1B680D

Copyright (C) 1986 PIL Software Systems                   45

Appendix A                                Terminal Emulation

Televideo 900 Series

ProComm  supports  the   standard   Televideo   900   series

functions.  Supported  functions include: full/half  duplex,

program function keys, cursor control, erase functions,  and

full  display attributes.  The  main  difference between the

910/920 and the 925/950 emulations is in the codes generated

for cursor down and in control codes for display attributes.

The special graphics character set  and user loadable status

line are supported.

ProComm KEYS     TELEVIDEO 9xx FUNCTION    TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Backtab ........ Reverse Tab ........................ 0x1B49

Insert ......... Insert Character ................... 0x1B51

Ctrl-Home ...... Insert Line ........................ 0x1B45

Delete ......... Delete Character ................... 0x1B57

Ctrl-PgUp ...... Delete Line ........................ 0x1B52

Home ........... Home Cursor .......................... 0x1E

Cursor Up ...... Cursor Up ............................ 0x1B

Cursor Down .... Cursor Down (910/920) ................ 0x0A

Cursor Down (925/950) ................ 0x16

Cursor Left .... Cursor Left .......................... 0x18

Cursor Right ... Cursor Right ......................... 0x1C

Ctrl-PgDn ...... Clear Screen ......................... 0x1A

End ............ Line Erase ......................... 0x1B54

Ctrl-End ....... Page Erase ......................... 0x1B59

F1 ............. Function 1 (F1) .................. 0x01400D

F2 ............. Function 2 (F2) .................. 0x01410D

F3 ............. Function 3 (F3) .................. 0x01420D

F4 ............. Function 4 (F4) .................. 0x01430D

F5 ............. Function 5 (F5) .................. 0x01440D

F6 ............. Function 6 (F6) .................. 0x01450D

F7 ............. Function 7 (F7) .................. 0x01460D

F8 ............. Function 8 (F8) .................. 0x01470D

F9 ............. Function 9 (F9) .................. 0x01480D

F10 ............ Function 10 (F10) ................ 0x01490D

Shift-F1 ....... Function 11 (F11) ................ 0x014A0D

Shift-F2 ....... FUNCT .......................... *see below

Shift-F3 ....... Shift Line Erase ................... 0x1B74

Shift-F4 ....... Shift Page Erase ................... 0x1B79

Shift-F5 ....... Shift Line Insert .................. 0x1B4E

Shift-F6 ....... Shift Line Delete .................. 0x1B4F

Shift-F7 ....... Shift Character Insert ............. 0x1B71

Shift-F8 ....... Shift Character Delete ............. 0x1B72

*  To emulate the  Televideo FUNCT key operation, press  and

release Shift-F2, then press and release the key you wish to

use in  in  conjunction  with  FUNCT.   This  will send  the

standard 3 byte FUNCT sequence: <SOH> <key pressed> <CR>.

Copyright (C) 1986 PIL Software Systems                   46

Appendix A                                Terminal Emulation

Digital Equipment Corporation VT-52

ProComm  supports  the   standard  model   VT-52  functions.

Supported  functions   include:   full/half  duplex,  keypad

application mode, cursor control, erase  functions, and full

display  attributes. ProComm  responds  to the  the Identify

command (ESC  Z) with ESC/Z.  (Standard VT-52 identification

sequence). Keypad Application Mode  functions are mapped  to

ProComm function keys as shown in the chart below.

ProComm KEYS     DEC VT-52 FUNCTION        TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Delete ......... Character Delete ..................... 0x7F

Home ........... Home Cursor ........................ 0x1B48

Cursor Up ...... Cursor Up .......................... 0x1B41

Cursor Down .... Cursor Down ........................ 0x1B42

Cursor Left .... Cursor Left ........................ 0x1B44

Cursor Right ... Cursor Right ....................... 0x1B43

Ctrl-PgDn ...... Clear screen ................... 0x1B481B4A

End ............ Erase end of line .................. 0x1B4B

F1 ............. Keypad Application mode 1 ........ 0x1B3F71

F2 ............. Keypad Application mode 2 ........ 0x1B3F72

F3 ............. Keypad Application mode 3 ........ 0x1B3F73

F4 ............. Keypad Application mode 4 ........ 0x1B3F74

F5 ............. Keypad Application mode 5 ........ 0x1B3F75

F6 ............. Keypad Application mode 6 ........ 0x1B3F76

F7 ............. Keypad Application mode 7 ........ 0x1B3F77

F8 ............. Keypad Application mode 8 ........ 0x1B3F78

F9 ............. Keypad Application mode 9 ........ 0x1B3F79

F10 ............ Keypad Application mode 0 ........ 0x1B3F70

Shift-F1 ....... Program Function 1 (PF1) ........... 0x1B50

Shift-F2 ....... Program Function 2 (PF2) ........... 0x1B51

Shift-F3 ....... Program Function 3 (PF3) ........... 0x1B52

Shift-F4 ....... Program Function 4 (PF4) ........... 0x1B53

Shift-F5 ....... Keypad Application mode DASH ..... 0x1B3F6D

Shift-F6 ....... Keypad Application mode COMMA .... 0x1B3F6C

Shift-F7 ....... Keypad Application mode PERIOD ... 0x1B3F6E

Shift-F8 ....... Keypad Application mode ENTER .... 0x1B3F4D

Copyright (C) 1986 PIL Software Systems                   47

Appendix A                                Terminal Emulation

Lear Sieglar ADM 3/5

ProComm  supports  the  standard  ADM 3/5  series functions.

Supported   functions   include:   full/half   duplex,  full

character set, erase functions, and cursor control.

ProComm KEYS     ADM 3/5 FUNCTION          TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ..................... 0x1B49

Backtab ........ Reverse Tab ........................ 0x1B51

Home ........... Home Cursor .......................... 0x1E

Cursor Up ...... Cursor Up ............................ 0x1B

Cursor Down .... Cursor Down .......................... 0x0A

Cursor Left .... Cursor Left .......................... 0x18

Cursor Right ... Cursor Right ......................... 0x1C

Ctrl-PgDn ...... Clear screen ......................... 0x1A

End ............ Erase end of line .................. 0x1B54

Copyright (C) 1986 PIL Software Systems                   48

Appendix A                                Terminal Emulation

Heath/Zenith 19

ProComm  supports the  standard Heath/Zenith  19  functions.

Supported   functions   include:   full/half  duplex,   full

character  set,  program  function  keys,  erase  functions,

cursor control, and display attributes.

ProComm KEYS     H-19 FUNCTION             TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Home ........... Home Cursor .................... 0x1B595F5F

Cursor Up ...... Cursor Up .......................... 0x1B41

Cursor Down .... Cursor Down ........................ 0x1B42

Cursor Left .... Cursor Left ........................ 0x1B44

Cursor Right ... Cursor Right ....................... 0x1B43

Ctrl-PgDn ...... Clear screen ....................... 0x1B4C

End ............ Erase end of line .................. 0x1B45

Ctrl-Home ...... Insert Line ........................ 0x1B4C

Ctrl-PgUp ...... Delete Line ........................ 0x1B4D

F1 ............. Program Function 1 (PF1) ........... 0x1B53

F2 ............. Program Function 2 (PF2) ........... 0x1B54

F3 ............. Program Function 3 (PF3) ........... 0x1B55

F4 ............. Program Function 4 (PF4) ........... 0x1B56

F5 ............. Program Function 5 (PF5) ........... 0x1B57

F6 ............. Program Function 6 (PF6) ........... 0x1B50

F7 ............. Program Function 7 (PF7) ........... 0x1B51

F8 ............. Program Function 8 (PF8) ........... 0x1B52

F9 ............. Program Function 9 (PF9) ........... 0x1B30

F10 ............ Program Function 10 (PF10) ......... 0x1B31

Copyright (C) 1986 PIL Software Systems                   49

Appendix A                                Terminal Emulation

ADDS Viewpoint

ProComm  supports  the  standard  ADDS Viewpoint  functions.

Supported  functions   include:   full/half  duplex,   erase

functions,   inset/delete  functions,  cursor  control,  and

display attributes.

ProComm KEYS     ADDS FUNCTION             TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Home ........... Home Cursor .................... 0x1B595F5F

Cursor Up ...... Cursor Up ............................ 0x01

Cursor Down .... Cursor Down .......................... 0x0A

Cursor Left .... Cursor Left .......................... 0x15

Cursor Right ... Cursor Right ......................... 0x06

Ctrl-PgDn ...... Clear screen ......................... 0x0C

End ............ Erase end of line .................. 0x1B4B

Ctrl-End ....... Erase end of screen ................ 0x1B6B

Ins ............ Insert Character ................... 0x1B46

Ctrl-Home ...... Insert Line ........................ 0x1B4D

Del ............ Delete Character ................... 0x1B45

Ctrl-PgUp ...... Delete Line ........................ 0x1B6C

F1 ............. Function 1 (F1) .................. 0x02310D

F2 ............. Function 2 (F2) .................. 0x02320D

F3 ............. Function 3 (F3) .................. 0x02330D

F4 ............. Function 4 (F4) .................. 0x02340D

F5 ............. Function 5 (F5) .................. 0x02350D

F6 ............. Function 6 (F6) .................. 0x02360D

F7 ............. Function 7 (F7) .................. 0x02370D

F8 ............. Function 8 (F8) .................. 0x02380D

Shift F1 ....... Shift Function 1 (F1) ............ 0x02390D

Shift F2 ....... Shift Function 2 (F2) ................ 0x3A

Shift F3 ....... Shift Function 3 (F3) ................ 0x3B

Shift F4 ....... Shift Function 4 (F4) ............ 0x3C2D2D

Shift F5 ....... Shift Function 5 (F5) ................ 0x3D

Shift F6 ....... Shift Function 6 (F6) ............ 0x2D2D3E

Shift F7 ....... Shift Function 7 (F7) ................ 0x3F

Shift F8 ....... Shift Function 8 (F8) ................ 0x40

Copyright (C) 1986 PIL Software Systems                   50

Appendix A                                Terminal Emulation

WYSE 100

ProComm  supports  the  standard  WYSE functions.  Supported

functions   include:  full/half  duplex,  erase   functions,

inset/delete   functions,   cursor  control,   and   display

attributes.

ProComm KEYS     WYSE 100 FUNCTION         TRANSMITTED CODES

------------------------------------------------------------

Tab ............ Horizontal Tab ....................... 0x09

Backtab ........ Reverse Tab ........................ 0x1B49

Insert ......... Insert Character ................... 0x1B51

Ctrl-Home ...... Insert Line ........................ 0x1B45

Delete ......... Delete Character ..................... 0x7F

Ctrl-PgUp ...... Delete Line ........................ 0x1B52

Home ........... Home Cursor .......................... 0x1E

Cursor Up ...... Cursor Up ............................ 0x1B

Cursor Down .... Cursor Down .......................... 0x0A

Cursor Left .... Cursor Left .......................... 0x18

Cursor Right ... Cursor Right ......................... 0x1C

Ctrl-PgDn ...... Clear Screen ......................... 0x1A

End ............ Line Erase ......................... 0x1B54

Ctrl-End ....... Page Erase ......................... 0x1B59

F1 ............. Function 1 (F1) .................. 0x01400D

F2 ............. Function 2 (F2) .................. 0x01410D

F3 ............. Function 3 (F3) .................. 0x01420D

F4 ............. Function 4 (F4) .................. 0x01430D

F5 ............. Function 5 (F5) .................. 0x01440D

F6 ............. Function 6 (F6) .................. 0x01450D

F7 ............. Function 7 (F7) .................. 0x01460D

F8 ............. Function 8 (F8) .................. 0x01470D

Shift F1 ....... Shift Function 1 (F1) ............ 0x01480D

Shift F2 ....... Shift Function 2 (F2) ............ 0x01490D

Shift F3 ....... Shift Function 3 (F3) ............ 0x014A0D

Shift F4 ....... Shift Function 4 (F4) ............ 0x014B0D

Shift F5 ....... Shift Function 5 (F5) ............ 0x014C0D

Shift F6 ....... Shift Function 6 (F6) ............ 0x014D0D

Shift F7 ....... Shift Function 7 (F7) ............ 0x014E0D

Shift F8 ....... Shift Function 8 (F8) ............ 0x014F0D

Copyright (C) 1986 PIL Software Systems                   51

Appendix A                                Terminal Emulation

ANSI-BBS

This  is  the recommended emulation  for use  with  bulletin

board systems that use ANSI graphics and color. the ANSI-BBS

mode processes the ANSI  codes  given  in  the DOS technical

reference manual, and behaves  in the same manner as the DOS

ANSI.SYS device driver.  (ProComm does not use the  ANSI.SYS

driver  itself.)  This emulation  is  similar to  the VT-100

emulation, but  the VT-100  emulation handles  normal, bold,

and reverse video in a different manner. The VT-100 keyboard

mapping is used with this emulation.

Copyright (C) 1986 PIL Software Systems                   52

Appendix B                                  ProComm Commands

COMMAND REFERENCE GUIDE

MAJOR FUNCTIONS

Dialing Directory .......... Alt-D

Automatic Redial............ Alt-R

Keyboard Macros ............ Alt-M

Modem Parameters ........... Alt-P

Translate Table ............ Alt-W

Editor ..................... Alt-A

Exit ....................... Alt-X

DOS Gateway ............... Alt-F4

Command File .............. Alt-F5

Redisplay ................. Alt-F6

UTILITY FUNCTIONS

Program Info ............... Alt-I

Setup screen ............... Alt-S

Kermit server commands ......Alt-K

Change Directory ........... Alt-B

Clear Screen ............... Alt-C

Toggle Duplex .............. Alt-E

Hang Up Phone .............. Alt-H

Elapsed Time ............... Alt-T

Print On/Off ............... Alt-L

Set Colors ................. Alt-Z

Toggle CR - CR/LF ......... Alt-F3

Break Key ............. Ctrl-Break

FILE FUNCTIONS

Send files .................. PgUp

Receive files ............... PgDn

File Directory ............. Alt-F

View a file ................ Alt-V

Screen Dump ................ Alt-G

Log Open/Close toggle ..... Alt-F1

Log hold .................. Alt-F2

Copyright (C) 1986 PIL Software Systems                   53

Appendix C                                     Command Files

COMMAND FILE COMMANDS (Alphabetical by Command)

Command Form                    Command Function

------------------------------------------------------------

%A                              Sound alarm.

%B"drive and/or dir"            Change default drive and/or

directory.

%C"dd entry"                    Call dialing directory entry

with long distance code.

%D"DOS command"                 Execute DOS commands.

%E0                             Emulate IBM 3101.

%E1                             Emulate DEC VT100.

%E2                             Emulate Televideo 912/920.

%E3                             Emulate Televideo 925/950.

%E4                             Emulate DEC VT52.

%E5                             Emulate ADM3/5.

%E6                             Emulate Heath 19.

%E7                             ANSI-BBS.

%E8                             ADDS Viewpoint.

%E9                             Wyse 100

%F"x"                           Change comm parameters (see

manual).

%G                              Screen dump to PROCOMM.IMG.

%H                              Hang-up the phone.

%I"string to wait for"          Wait for incoming string.

%Jx                             Send ALT key sequence.

%KBB                            Kermit binary mode.

%KBT                            Kermit text mode.

%KF                             Send Kermit Finish server

command.

%KL                             Send Kermit Logout server

command.

%LC                             Close log file.

%LH                             Hold log file.

%LO"logname"                    Open log file.

%LPC                            Close printer logging.

%LPO                            Open printer logging.

%LR                             Resume log file.

%M"message"                     Display message on screen.

%NY"prompt"                     Prompt for, get and send

user input.

%NN"prompt"                     Prompt for, get and send

user input (masked).

%OB                             Use BIOS for screen writes.

%OD                             Use direct screen writes.

%P"seconds to look"             Seconds to look for incoming string.

%Q                              Quit ProComm.

%R"redial delay"                Reset redial delay time.

%SAY                            Alarm on.

%SAN                            Alarm off.

%SN                             Sound off.

%ST"time in seconds"            Set alarm time.

%SY                             Sound on.

Copyright (C) 1986 PIL Software Systems                   54

Appendix C                                     Command Files

%T"string"                      Transmit string out port.

%U1                             Clear screen.

%U2                             CR as CR.

%U3                             CR as CR/LF.

%U4                             Full duplex.

%U5                             Half duplex.

%U6                             Break.

%U7                             Line wrap on.

%U8                             Line wrap off.

%U9                             Scroll on.

%U0                             Scroll off.

%UBN                            BS non-destructive

%UBY                            BS destructive.

%UHN                            XON/XOFF handshake off.

%UHY                            XON/XOFF handshake on.

%UXN                            XMODEM normal mode.

%UXR                            XMODEM relaxed mode.

%W"seconds to wait"             Wait for "seconds" seconds.

%XDK                            Download Kermit.

%XDM                            Download MODEM7.

%XDT                            Download Telink.

%XDX"filespec"                  Download XMODEM.

%XDY"filespec"                  Download YMODEM.

%XDZ                            Download YMODEM batch.

%XUA"fielspec"                  Upload ASCII.

%XUK"filespec"                  Upload Kermit.

%XUM"filespec"                  Upload MODEM7.

%XUT"filespec"                  Upload Telink.

%XUX"filespec"                  Upload XMODEM.

%XUY"filespec"                  Upload YMODEM.

%XUZ"filespec"                  Upload YMODEM batch.

%YEN                            ASCII u/l echo off.

%YEY                            ASCII u/l echo on.

%YP"pace count"                 Set ASCII upload pace count.

%YSN                            ASCII u/l don't strip LF

%YSY                            ASCII u/l strip LF

%ZY                             Translate table on.

%ZN                             Translate table off.

All  commands  start  with '%'.  The  command letter(s) must

immediately follow the '%', and may be upper  or lower case.

All arguments  must be enclosed in  quotes ("")or tick marks

(''),  and must immediately follow the command letters.  All

other characters in the file are ignored, and may be used as

comments. ProComm looks  for an  extension of  .CMD  when it

lists available command files.

When using the  'I'  command, the strings must match exactly

in case.  Use  the  'P' command to  set the  amount of  time

ProComm waits for the  incoming character string to be found

(default is 30).

Strings to be transmitted ('T'  command) use the translation

Copyright (C) 1986 PIL Software Systems                   55

Appendix C                                     Command Files

conventions  described  in  the  section  on  output  string

translation.

Copyright (C) 1986 PIL Software Systems                   56

Appendix D                                  Utility Programs

Timed Execution Facility (TEF)

The  ProComm  Timed  Execution  Facility  is  a  stand alone

program that allows execution of a ProComm command file at a

predetermined time.

TEF is started  from  the  DOS prompt by typing the command:

TEF

TEF will prompt you for the  name of a ProComm  command file

and the time to start its execution. TEF will wait until the

specified  time and then load and execute ProComm passing in

the name of the command file.

The time must be entered  in military format.  The hour must

be 0-23 and the minutes  must be 0-59.  Pressing ESC at  any

time  will abort TEF. Pressing F1 will allow  you  to change

the command  file and execution time. Pressing F2 will blank

the screen.  (Pressing  any other key  will turn  the screen

back on).

Dialing Directory Conversion Utility

ProComm is distributed with a  dialing directory  conversion

program called CONVERT.EXE. Use CONVERT to change PC-TALK or

QMODEM dialing  directories to  ProComm format.  To execute,

type: CONVERT on  the  DOS command line and  then follow the

prompts.

NOTE: When converting QMODEM  directories only the first 100

entries will  be  converted. In addition, the modem  command

will not be converted and will default to ATDT.

Copyright (C) 1986 PIL Software Systems                   57

Appendix E                              Answers to Questions

Question:

ALT-H doesn't work with my modem. What's wrong?

Answer:

ProComm first attempts to  hang up by dropping  the DTR line

to the modem.  Your modem must monitor the DTR line for this

to work. Users of Hayes external modems must have switch one

in  the  open  (up)  position.  Owners  of  the newer  Hayes

internals  must have  a  jumper correctly set.  Consult your

modem  manual  for  further  details. If  your configuration

prevents the use of DTR in hanging up, ProComm can also send

a hangup string. After  attempting  to  drop DTR the program

checks carrier detect to  see if you are still online. If so

it will  send  the  hangup string you have  specified in the

setup screen.

Question:

When  using  the redial facility,  or  making a call  from a

command file, the program doesn't realize  when I've  made a

connection.

Answer:

Make sure that ProComm's modem  connect string  is correctly

defined  for  the message your  modem  returns  for  carrier

detect. The ProComm default is "CONNECT".

Question:

I  sometimes  have  trouble opening  files  for  logging  or

downloading.

Answer:

You  may  need  to  increase  the  FILES=  parameter  in your

CONFIG.SYS  file.   We  suggest  using  at  least   FILES=15.

Consult your DOS manual for more details.

Question:

I can't get the  dialing directory  to call  using  the long

distance code I've put in the number.

Answer:

Specify the long distance code to be used  when  you make to

call  itself,  not  when  you  enter  the  number  into  the

directory.  For instance, say "#37"  at the "==>"  prompt to

Copyright (C) 1986 PIL Software Systems                   58

Appendix E                              Answers to Questions

dial entry  number  37 using  the  '#' long  distance  code.

Entering a long distance code as part of the number will not

work.

Question:

I'm having trouble getting ProComm to work with my PCjr. Any

ideas?

Answer:

The  PCjr  is  somewhat  different  in  the  way it  handles

communications. A solution  that has worked  for a number of

users is  this: Call our  BBS  and  download a  file  called

COMSWAP.ARC.  After  un-arcing  the  file you  should find 2

programs  called COMSWAP.COM  and  COMREST.COM.  Run COMSWAP

before  running ProComm  and  run  COMREST  after. Then  use

ProComm set to COM2:.

Question:

When  I call your  bulletin board my modem connects but then

nothing else happens. What's wrong?

Answer:

The Fido  BBS software, and a  number of other BBS programs,

require some input so that the modems can determine the baud

rate and  start their handshake. After  connecting, send the

remote several carriage returns.  If you  are calling a Fido

at 2400 baud you should send several SPACES instead.

Question:

When I'm using a mutli-tasking operating system and  running

ProComm in  the background, window displays bleed through to

the active partition.

Answer:

Go into the  general setup screen and set ProComm to use the

BIOS screen  write method.  It is  much slower but will  not

cause the bleed through that you are experiencing.

Question:

When I try to use the DOS gateway it says 'Command processor

not found'. What does this mean?

Answer:

Copyright (C) 1986 PIL Software Systems                   59

Appendix E                              Answers to Questions

In order to  use the gateway, ProComm must load  a secondary

copy  of the  command processor  (COMMAND.COM). It uses  the

environment  variable  COMSPEC  to  determine  the name  and

location of the  command processor. COMSPEC is  set when you

boot  to the drive, directory and program  you  booted from.

Problems can arise if you boot off a floppy, and then change

the floppy in the boot disk drive.  To use  the gateway, and

the ALT-F function, be sure that COMSPEC is set and that the

command processor is where COMSPEC says it is.

Question:

I  can't  use  the help screen because MultiLink  grabs  the

ALT-F10 keystroke before ProComm can get it.

Answer:

MultiLink provides  an  'escape'  keystroke  to  avoid  that

problem. Simply  press  ALT-F9 before  pressing ALT-F10  and

MultiLink will let the ALT-F10 through to ProComm.

Copyright (C) 1986 PIL Software Systems                   60

Appendix F                          Technical Specifications

ProComm Technical Specifications

Receive buffer size ...................  2048 Bytes

Redisplay buffer size .................. 4000 Bytes

Baud rates supported .......................... 300

......................... 1200

......................... 2400

......................... 4800

......................... 9600

........................ 19200

File transfer protocols ..................... ASCII

.................... XMODEM

.................... TELINK

.................... YMODEM

.................... MODEM7

.................... KERMIT

Serial ports supported ............ COM1 0x3F8 IRQ4

............ COM2 0x2F8 IRQ3

Video buffer addresses supported .... COLOR 0xB8000

..... MONO 0xB0000

Interrupt vectors used ....................... 0x0B

....................... 0x0C

....................... 0x1B

....................... 0x23

Copyright (C) 1986 PIL Software Systems                   61

Appendix G                                           Invoice

Remit to:          PIL Software Systems              Invoice

PO Box 1471

Columbia, MO 65205

Sold to:                       Ship to:

--------------------           --------------------

--------------------           --------------------

--------------------           --------------------

============================================================

Date:                          PO #:

============================================================

Quantity       Description                  Unit    Extended

------------------------------------------------------------

| ProComm v2.2                 | $25.00 | $

|                              |        |

|                              |        |

|                              |        |

|                              |        |

|                              |        |

------------------------------------------------------------

Subtotal: $

(MO residents please add 4.3% sales tax)       Tax: $

Total: $

Copyright (C) 1986 PIL Software Systems                   62

ProComm 2.2                                            Index

INDEX                          ALT-F4  36

ALT-F5  27

ALT-F6  38

ALT-G  37

%                              ALT-H  14, 40

%A  29                       ALT-I  39

%B  29                       ALT-K  26

%C  29                       ALT-L  37

%D  29                       ALT-M  36

%E  29                       ALT-P  10

%F  30                       ALT-R  23

%G  30                       ALT-S  12

%H  30                       ALT-T  39

%I  30                       ALT-V  39

%J  31                       ALT-W  27

%K  31                       ALT-X  41

%L  31                       ALT-Z  38

%M  32                       ANSI-BBS  52

%N  32                       ASCII file xfer  24

%O  32                       auto redial

%P  32                         See redial

%Q  32                       AUTOEXEC.BAT  7

%R  32

%S  32                     B

%T  33                       bar

%U  33                         See vertical bar

%W  34                       BIOS screen write  17,

%X  34                           59

%Y  35                       break  41

%Z  36                       BS interpretation  15

/                            C

/B  8                        caret (^)  11

/F  8, 27                    carrier detect mask

/M  8                            14, 40

/S  8                        CD mask

See carrier detect

A                                mask

aborted files  19            changes to previous  1

address of PIL  2            clear screen  40

ADDS Viewpoint  50           CMD files

alarm sound  17                See script files

alarm time  18               colors

ALT-A  37                      See setting colors

ALT-B  39                    command files

ALT-C  40                      See script files

ALT-D  19                    command line options  8

ALT-E  40                    command processor  60

ALT-F  39                    command reference  53

ALT-F1  37                   commercial use  ii

ALT-F10  9, 60               communications param  9

ALT-F2  37                   CompuServe  18, 25

ALT-F3  40

Copyright (C) 1986 PIL Software Systems             Index 63

ProComm 2.2                                            Index

COMSPEC  36, 39, 60              24

CONFIG.SYS  6                file transfer  23

connect string  13, 58       file viewing  39

control characters  11       file xfer protocols  24

CONVERT  57                  FILES=  6, 58

copying  ii

copying the program  2     G

corporate license  ii        general setup  16

CR interpretation  15

CR translation char  10    H

CR-CR/LF  15, 40             handshake  15

CRC                          hang-up  40, 58

See XMODEM CRC             hang-up string  13

CTRL translate char  11      Heath/Zenith 19  49

CTRL-BREAK  41               help screen  9

D                            I

data terminal ready          IBM 3101  45

See DTR                    information screen  8

DEC VT-100  43               invoice  2, 62

DEC VT-100 kb map  44

DEC VT-52  47              K

default directory  39        Kermit file xfer  26

default drive  39            Kermit setup  16

dialing  22                  keyboard macros  36

dialing command  13          keystroke changes  1

dialing directory  19

dialing, manual  22        L

direct screen write  17      license  ii

distribution  4              line wrap  16

DOS commands  29             log file  17

DOS gateway  36, 59          logging, file  37

download, ASCII  24          logging, printer  37

downloaded files  19         long distance codes

downloading files  24            21, 58

DTR  40, 58, 58              LS ADM 3  48

duplex  15, 40

M

E                              machine requirements  6

echo                         making a call  22

See duplex                 manual dialing

editor  16, 37                 See dialing, manual

elapsed time  39             modem command  21

ESC translate char  10       modem init string  12

exclamation point (!         modem pause  14, 23

10                       modem pause char  11

modem setup  12

F                              MODEM7 file xfer  25

Fido  59                     multi-tasking  59

file directory  39           MultiLink  60

file logging

See logging, file        N

file name, duplicate

Copyright (C) 1986 PIL Software Systems             Index 64

ProComm 2.2                                            Index

no connect string  13          See Kermit file xfer

sound effects  17

O                              Source, The  5, 26

output string transl         stripping chars  27

10                       support BBS  2, 4

overlays  1

T

P                              TCOMM BBS  26

parameter file chang  1      TEF  57

pause delay  14              Televideo 900 series

pause translate cha-             46

11                       Telink file xfer  26

PC-HOST BBS  26              terminal emulation  9,

PCjr  59                         15, 42

PgDn  24                     terminal mode  9

PgUp  23                     terminal setup  15

print directory  22          tilde (~)  11

printer logging              timeout delay  14, 23

See logging, printer       translate CR char  18

problem reporting  4         translate CTRL char  19

ProComm environment  7       translate ESC char  19

ProComm files  7             translate pause char

PROCOMM.IMG  37                  18

product support  4           translate table  17, 27

PROFILE.CMD  28

program description  6     U

program information  39      upload echo  18

upload pacing  18

R                              upload strip LF  18

redial  13, 22               upload, ASCII  24

redispay buffer  38          uploading files  23

registration  ii, 2          user-supported softw  3

S                            V

screen dump  37              vertical bar (|)  10

screen dump file  17

screen write method  17    W

script cmd arguments         windows  18

28                       Wyse 100  51

script cmd reference

54                     X

script cmd syntax  28        XMODEM CRC  25

script file changes  1       XMODEM file xfer  25

script files  27             XMODEM mode  18

scrolling  16                XMODEM relaxed  18, 25

SET command  7               XON/XOFF  15

setting colors  38

setup options  11          Y

site license  ii             YMODEM file xfer  25

sliding windows

Copyright (C) 1986 PIL Software Systems             Index 65