ADDENDUM FOR THE DATAREMOTE CDS-902X AND 905X USER GUIDES   8-1-02

SMS FOR THE CDS-902X AND CDS-905X SERIES CELLULAR MODEMS

To initiate from 1 to 20 SMS messages:

AT+CMGS=(enter e-mail address here), enter

This will switch the modem from data mode to SMS mode and enter the e-mail address.  Note that after a 15 to 20 second pause, D6, the FAM MODE-AUX LED, will start blinking signifying that you have entered SMS mode and a > prompt will appear on the screen.  At the < prompt enter the desired SMS message.  The message length may be up to 170 characters/bytes long.  This total includes the previously entered e-mail address.

After delivery of the SMS message you will see the prompt:

+CMGS:11

OK

To send more messages re-enter:

AT+CMGS=<enter e-mail address or phone number here>

<enter>

You can deliver up to 20 SMS messages in one session without going back to data mode.  After prolonged inactivity of approximately 120 seconds, the modem will automatically switch back to data mode.  This transition normally takes about 20 seconds.

To leave the SMS mode and return to data mode, enter

AT, <enter>

There will be a 20 second delay during mode reset before you return to data mode and receive the OK prompt.

The following are the commands that may be used in conjunction with SMS.  They are based on the standard GSM command set.

+CMGD	Delete message from storage

AT+CMGD=<index><CR>

This command deletes the stored message at location <index>.  Note that the index values change whenever a new message is received, sent or deleted.  A valid way to perform a series of deletes is to delete the highest numbered index first.

+CMGL	Read several messages from storage

AT+CMGL=<stat><CR>

+CMGL: <index>,<stat>,<oa/da>,,<scts><CR><LF>

<data><CR><LF>[…]

This command will read back all messages in storage that match the <stat> parameter.  Response will be generated for each stored message.

+CMGR	Read a single message from storage

AT+CMGR=<index><CR>

+CMGR: <index>,<stat>,<oa/da>,,<scts><CR><LF><data><CR><LF>

This command reads back the message at location <index>.

+CMGS	Send a message (directly)

AT+CMGS=<da><CR>

><data><^Z/ESC>

This command sends the message <data> to the telephone number or URL <da>.

+CMGW	Write a message to storage

AT+CMGW<CR>

><data><^Z/ESC>

+CMGW:<index>

This command stores the message <data>.  No address <da> information is saved with the message.

+CMSS	Send a message from storage

AT+CMSS=<index>,<da><CR>

This command sends the message stored at location <index> to the telephone number or URL <da>.

# SYMBOLS

<index>	Storage location (1-30)

1-10:    REC UNREAD and RED READ messages

11-20:  STO SENT messages

21-30:  STO UNSENT messages

<da>		Destination address

Either a telephone number or internet URL

<oa>		Origin address

Either a telephone number or internet URL

<scts>		Service center time stamp

TIME/DATE

<data>		Message text content

ASCII content terminated with any control character

CMGL

<stat>		0 – Received unread message		“REC UNREAD”

1 – Received read message		“REC READ"

2 – Stored unsent message		“STO UNSENT”

3 – Stored send message			“STO SENT”

4 – All messages 				“ALL”

## “FORCED” SMS MESSAGING / CRY-OUT ALARMS

To enter SMS data for remote “forced” message transmission follow the following procedure:

- Enter the configuration menu (Type control-WWW within 2 seconds)

- Select option “I” (Program Alarm Number)

- Enter “0” to disable the alarm

- Enter the new alarm number: (the e-mail address)  (up to 58 characters maximum)

- Select option “K” (Site Name)

- Enter SMS message (up to 74 characters maximum)

- Enter option “O” to exit the terminal mode

When P7, the I/O connector, circuit “3” is momentarily shorted for at least 2 seconds, the modem automatically switches to SMS mode and delivers the message, and then switches back to data mode.  Please note that there are time durations of approximately 20 seconds that occur while switching from data to SMS modes and back again.  (Refer to figure 1)

D6   I I I I I I

6  4  2

FIG. 1 I/O Circuit Numbers           5  3  1

Circuit 1…. Voice Dial Input

Circuit 2…. Data Call Input

Circuit 3…. SMS / Alarm Input

## Circuit 4-6…. Reserved for DRIP Command set in conjunction with DRM-9021 memory module. (CDS-905X)

## VOICE MODE FOR THE CDS-9022 AND CDS-9052 CELLULAR MODEMS

Using voice mode requires DataRemote headset model DRH-HDST for proper operation.  To initiate voice mode from data mode, enter:

AT

OK (response from modem)

AT+CDV<phone number you wish to dial>

You will hear a faint beep over the headset after a few seconds and then ringing.  When called party answers go ahead and talk.  If an extension number or other numerical input is required to complete the connection enter:

ATDT<desired number>

To end the conversation and return to data mode, enter:

ATH

If the landline side hangs up during or at the end of a voice call, the DataRemote modem will disconnect within 20 seconds and return to data mode automatically.